GET 311

Engineering Mathematics III

Topic Introductions and Departmental Application Questions

3 Units  |  Compulsory  |  LH 45

# Contents

# Course Overview

Credit: 3 Units | Status: C (Compulsory) | Contact: LH 45

This document is a study companion for GET 311. For each topic in the course outline it gives:

- a short introduction to the topic, and

- two application questions for each of the eight engineering departments that take the course (16 questions per topic, 128 in total).

## Learning Outcomes

By the end of the course, students should be able to:

- show a clear understanding of the course content, with broad knowledge of the areas covered;

- have a deep enough foundation to go on to advanced mathematical topics;

- develop simple algorithms and compute accurately;

- write simple proofs of theorems and apply them; and

- communicate mathematical knowledge well in speech, in writing and in groups.

## Course Contents

| # | Topic |
| --- | --- |
| 1 | Linear Algebra |
| 2 | Elements of Matrices, Determinants, Inverses of Matrices |
| 3 | Theory of Linear Equations; Eigenvalues and Eigenvectors |
| 4 | Analytical Geometry; Coordinate Transformation; Solid Geometry; Polar, Cylindrical and Spherical Coordinates |
| 5 | Elements of Functions of Several Variables; Surface Variables |
| 6 | Ordinary, Double, Triple, Line and Surface Integrals; Vector Calculus (Gradient, Flux, Curl) |
| 7 | Gauss's, Green's and Stokes' Theorems and Applications; Single-Valued and Multivalued Functions |
| 8 | Analytic Functions; Cauchy–Riemann Equations; Singularities and Zeros; Contour Integration; Bilinear Transformation |

## Applied Departments

- Agricultural Engineering

- Chemical Engineering

- Computer Engineering

- Civil Engineering

- Electrical and Electronics Engineering

- Food Engineering

- Mechanical Engineering

- Petroleum Engineering

## How to Use This Material

- Read the introduction to a topic before the lecture on it.

- Try the questions for your own department first, then do the others. Most of them use the same method in a different setting.

- The questions fit tutorials, group work (Learning Outcome 5) and small programming exercises in Python or MATLAB (Learning Outcome 3).

# Topic 1: Linear Algebra

## Introduction

Linear algebra studies vectors, vector spaces and linear transformations. A vector space is a set of objects that can be added together and multiplied by scalars under a fixed set of rules (axioms). Examples are ℝⁿ, the set of polynomials and the set of solutions of a linear differential equation. The main ideas are:

- Linear combination and span: the set of all vectors you can build from a given set by scaling and adding.

- Linear independence: no vector in the set can be written as a combination of the others.

- Basis and dimension: a basis is a minimal spanning set, and the number of vectors in it is the dimension of the space.

- Linear transformations: maps T with T(αu + βv) = αT(u) + βT(v). Every such map between finite-dimensional spaces can be written as a matrix.

- Inner product, norm and orthogonality: tools for measuring length and angle, projecting and finding least-squares approximations.

Engineering uses linear algebra constantly. Structural analysis, circuit networks, signal processing, computer graphics, process mass balances and reservoir simulation all reduce to linear models, which makes this topic the base for the rest of the course.

## Application Questions

### 1. Agricultural Engineering

- A fertilizer blend is made from three stock fertilizers with N-P-K compositions a = (20, 10, 10), b = (0, 20, 20) and c = (10, 0, 30) (percent by mass). Decide whether these three vectors are linearly independent. Can any target composition in ℝ³ be reached by blending them? Explain using the idea of span.

- Soil samples from a farm are described by the vector (moisture %, organic matter %, pH). Given s₁ = (22, 3.1, 6.2), s₂ = (18, 2.4, 5.8) and a reference sample r = (20, 2.8, 6.0), find the Euclidean distance of each sample from the reference and the angle between s₁ and s₂. Comment on how alike the samples are.

### 2. Chemical Engineering

- The combustion of methane, CH₄ + O₂ → CO₂ + H₂O, can be balanced by writing the atom counts (C, H, O) of each species as vectors and finding a non-trivial vector in the null space of the resulting matrix. Set up the vector equation and find the smallest integer stoichiometric coefficients.

- In a three-component mixture, the composition vectors of streams A, B and C are x_A = (0.5, 0.3, 0.2), x_B = (0.2, 0.5, 0.3) and x_C = (0.3, 0.2, 0.5). Show that the set {x_A, x_B, x_C} is a basis of ℝ³, and express a product stream x_P = (0.35, 0.35, 0.30) as a linear combination of them.

### 3. Computer Engineering

- A 2-D graphics engine rotates every point by θ = 30° and then scales it by 2 in the x-direction. Write each step as a linear transformation matrix, find the combined matrix, and apply it to the vertices of the unit square.

- In a basic document-search system, each document is a term-frequency vector. With d₁ = (3, 0, 1, 2) and d₂ = (1, 1, 0, 2), compute the cosine similarity between them. Write a short algorithm (pseudocode) that returns the document most similar to a query vector q.

### 4. Civil Engineering

- Three forces act at a truss joint: F₁ = (10, 0) kN, F₂ = (−5, 8) kN and F₃ = (a, b) kN. Use vector addition to find F₃ so the joint is in equilibrium. Are F₁ and F₂ linearly independent? What does that mean for the joint?

- A surveyor records displacement vectors between control points: AB = (120, 45, 2) m and BC = (−30, 80, −1) m. Find AC, its length, and the unit vector along AC. Find the projection of BC on AB.

### 5. Electrical and Electronics Engineering

- A three-phase balanced voltage set can be written as vectors in ℝ² (phasor components). Show that the phasors V∠0°, V∠−120° and V∠120° are linearly dependent, and that their sum is the zero vector.

- A discrete signal of 4 samples, x = (4, 2, 0, 2), is expanded in the orthogonal basis h₁ = (1, 1, 1, 1), h₂ = (1, 1, −1, −1), h₃ = (1, −1, 0, 0), h₄ = (0, 0, 1, −1) (a Haar-type basis). Check that the basis is orthogonal, then find the coefficients of x using inner products.

### 6. Food Engineering

- A food product is formulated from three ingredients whose (protein, fat, carbohydrate) contents in g per 100 g are (12, 2, 70), (25, 15, 5) and (1, 0, 95). Decide whether any nutritional profile can be made by mixing them (span). Find the mass of each ingredient needed to supply exactly (10, 3, 75) g of protein, fat and carbohydrate.

- The colour of a juice is measured in (L, a, b*) colour space. The target is (45, 20, 30) and a batch measures (42, 24, 27). Compute the colour difference ΔE as a Euclidean norm, and the unit vector giving the direction of the colour error.

### 7. Mechanical Engineering

- A force F = (3, −2, 5) N acts at the point r = (1, 2, 0) m. Compute the moment M = r × F about the origin, and check that M is orthogonal to both r and F using the dot product.

- The displacement field of a deformed plate is modelled by a linear map T(x, y) = (x + 0.02y, 0.01x + y). Write the matrix of T, find the image of the unit vectors, and decide whether T is invertible.

### 8. Petroleum Engineering

- The composition of a reservoir fluid is recorded as mole fractions of (C₁, C₂, C₃, C₄₊). Samples from wells W₁ and W₂ are (0.70, 0.12, 0.08, 0.10) and (0.60, 0.15, 0.10, 0.15). Find the composition of a blend with 40 % of W₁ and 60 % of W₂, and the Euclidean distance between the two well compositions.

- A deviated well path is given by survey displacement vectors d₁ = (0, 0, −500) m, d₂ = (150, 100, −400) m and d₃ = (300, 200, −200) m. Find the total displacement vector to the target and the horizontal departure. Decide whether the three vectors are linearly independent.

# Topic 2: Elements of Matrices, Determinants and Inverses of Matrices

## Introduction

A matrix is a rectangular array of numbers set out in rows and columns. It is a compact way to store data and to represent linear transformations. The key ideas are:

- Types of matrices: square, diagonal, identity, triangular, symmetric, skew-symmetric, orthogonal and singular.

- Matrix algebra: addition, scalar multiplication, matrix multiplication (associative but not commutative) and the transpose.

- Determinant: a scalar attached to a square matrix, found by cofactor expansion or row reduction. It measures how much the transformation scales volume. A matrix is invertible exactly when det(A) ≠ 0.

- Properties of determinants: the effect of row operations, det(AB) = det(A)det(B), and det(Aᵀ) = det(A).

- Inverse of a matrix: A⁻¹ satisfies AA⁻¹ = I. It can be found with the adjoint formula A⁻¹ = adj(A)/det(A) or with Gauss–Jordan elimination.

- Rank: the number of linearly independent rows or columns.

Engineers use matrices to store stiffness, conductance, transition and input–output data, and use inverses and determinants to solve for unknowns, test stability and change coordinates.

## Application Questions

### 1. Agricultural Engineering

- A farm makes three crops (maize, cassava, yam) over two seasons. The yield matrix Y (tonnes/ha) is 2 × 3 and the price vector p (₦/tonne) is 3 × 1. With Y = [[2.5, 12, 8], [3.0, 10, 9]] and p = (150 000, 40 000, 60 000)ᵀ, compute the revenue per hectare for each season using matrix multiplication.

- A tractor's hitch linkage gives the transformation matrix A = [[2, 1], [1, 3]] between input and output displacements. Find det(A) and A⁻¹, and explain what det(A) says about how the linkage changes the area of the motion region.

### 2. Chemical Engineering

- A mixer takes three feed streams, and the composition matrix C (rows = components, columns = streams) is [[0.6, 0.2, 0.1], [0.3, 0.5, 0.2], [0.1, 0.3, 0.7]]. Find C⁻¹ and use it to get the feed flow rates that give component product flows (50, 40, 60) kg/h.

- The rate constants of a reversible reaction network are placed in a 3 × 3 matrix K. Show that if the rows of K add to zero (mass conservation), then det(K) = 0. Explain why this makes K singular in physical terms.

### 3. Computer Engineering

- In a Hill cipher, a message is encrypted with the key matrix K = [[3, 3], [2, 5]] (mod 26). Find det(K) mod 26, check that K is invertible mod 26, and find the decryption matrix K⁻¹ mod 26.

- Homogeneous coordinates use 3 × 3 matrices for 2-D translation, rotation and scaling. Write the matrix that translates by (4, −2) and then rotates by 90°. Find its determinant and inverse, and explain why the determinant is 1.

### 4. Civil Engineering

- A two-member spring model of a structure has stiffness matrix K = [[k₁ + k₂, −k₂], [−k₂, k₂]] with k₁ = 200 kN/m and k₂ = 150 kN/m. Find K⁻¹ (the flexibility matrix) and the displacements under loads F = (10, 5)ᵀ kN.

- Show that the stiffness matrix of an unsupported beam element has determinant zero. Explain, using rank, why supports (boundary conditions) must be applied before the system can be solved.

### 5. Electrical and Electronics Engineering

- A two-port network has the impedance matrix Z = [[10, 4], [4, 6]] Ω. Find the admittance matrix Y = Z⁻¹ and the port currents when V = (20, 12)ᵀ V.

- Nodal analysis of a three-node resistive circuit gives the conductance matrix G = [[0.5, −0.2, 0], [−0.2, 0.7, −0.3], [0, −0.3, 0.4]] S. Compute det(G) by cofactor expansion and state whether a unique set of node voltages exists.

### 6. Food Engineering

- A bakery makes bread, cake and biscuits. The ingredient-use matrix (kg of flour, sugar and fat per batch) is A = [[5, 3, 2], [0.5, 2, 1], [0.3, 1.5, 1]]. With daily stock (80, 25, 18) kg, find A⁻¹ and the number of batches that uses all the stock exactly.

- The heat-transfer coefficients between three zones of a tunnel dryer form a symmetric matrix H. Given H = [[4, −1, 0], [−1, 4, −1], [0, −1, 4]], show that H is symmetric, compute det(H), and find H⁻¹.

### 7. Mechanical Engineering

- The rotation matrix about the z-axis is R(θ) = [[cos θ, −sin θ, 0], [sin θ, cos θ, 0], [0, 0, 1]]. Prove that R is orthogonal (RᵀR = I), that det R = 1, and that R⁻¹ = R(−θ).

- A two-degree-of-freedom spring–mass system has mass matrix M = diag(2, 1) kg and stiffness matrix K = [[300, −100], [−100, 100]] N/m. Compute M⁻¹K, which is needed for the vibration analysis in Topic 3.

### 8. Petroleum Engineering

- A finite-difference model of pressure in a 1-D reservoir with three blocks gives the transmissibility matrix T = [[2, −1, 0], [−1, 2, −1], [0, −1, 2]]. Find T⁻¹ and the block pressures when Tp = (1500, 0, 1200)ᵀ (suitably scaled).

- Three wells produce oil, gas and water. The production-rate matrix is P = [[500, 200, 50], [300, 400, 100], [200, 100, 300]] (units/day). Find det(P) and explain what a zero determinant would mean for back-calculating each well's contribution from total separator readings.

# Topic 3: Theory of Linear Equations; Eigenvalues and Eigenvectors

## Introduction

Systems of linear equations Ax = b are the most common computational task in engineering. The theory answers three questions:

- Existence and uniqueness (consistency): by the Rouché–Capelli theorem, a system has a solution if and only if rank(A) = rank([A | b]). The solution is unique when that rank equals the number of unknowns. Otherwise there are infinitely many solutions.

- Homogeneous systems: Ax = 0 always has the trivial solution, and has non-trivial solutions exactly when det(A) = 0.

- Solution methods: Cramer's rule, Gaussian and Gauss–Jordan elimination, LU decomposition, and iterative methods (Jacobi, Gauss–Seidel) for large sparse systems.

Eigenvalues and eigenvectors describe directions that a matrix only stretches: Av = λv. The eigenvalues are the roots of the characteristic equation det(A − λI) = 0. Important results include:

- the sum of the eigenvalues is trace(A) and their product is det(A);

- a symmetric matrix has real eigenvalues and orthogonal eigenvectors;

- diagonalisation A = PDP⁻¹, which makes powers and exponentials of A easy to compute;

- the Cayley–Hamilton theorem: every square matrix satisfies its own characteristic equation.

Eigen-analysis gives natural frequencies and vibration modes, the stability of dynamic systems, principal stresses, the steady states of Markov processes and the principal components of data.

## Application Questions

### 1. Agricultural Engineering

- An irrigation network has three canals with flows q₁, q₂, q₃ (m³/s) that satisfy q₁ + q₂ + q₃ = 12, 2q₁ − q₂ = 3 and q₂ − q₃ = 1. Use Gaussian elimination to find the flows, and check the result with Cramer's rule.

- A livestock population has calves, yearlings and adults. It is modelled by the Leslie matrix L = [[0, 0, 0.8], [0.6, 0, 0], [0, 0.9, 0.9]]. Find the dominant eigenvalue and its eigenvector. What do they say about long-term herd growth and the herd's age structure?

### 2. Chemical Engineering

- A system of three CSTRs in series with recycle gives the linear equations 5C₁ − C₃ = 20, −4C₁ + 5C₂ = 0 and −4C₂ + 5C₃ = 0 (concentrations in mol/L). Decide whether the system is consistent, then solve it using LU decomposition.

- Two first-order reactions A ⇌ B are modelled by dx/dt = Kx with K = [[−3, 1], [3, −1]]. Find the eigenvalues and eigenvectors of K, and use them to write the general solution. Interpret the zero eigenvalue as the equilibrium composition.

### 3. Computer Engineering

- Write an algorithm (pseudocode or Python) for Gauss–Seidel iteration, and use it for 3 iterations on 10x − y = 9, −x + 10y − 2z = 7, −2y + 10z = 6. State the convergence condition (diagonal dominance) and check it.

- A simplified PageRank for 3 web pages uses the column-stochastic link matrix M = [[0, 0.5, 0.5], [0.5, 0, 0.5], [0.5, 0.5, 0]]. Show that λ = 1 is an eigenvalue, and find the normalised eigenvector that gives the page ranks.

### 4. Civil Engineering

- The plane stress at a point is given by the tensor σ = [[80, 30], [30, 20]] MPa. Find the principal stresses (eigenvalues) and principal directions (eigenvectors), and give the angle of the principal plane.

- A three-storey shear building has mass matrix M = mI and stiffness matrix K = k[[2, −1, 0], [−1, 2, −1], [0, −1, 1]]. Set up the eigenvalue problem (K − ω²M)φ = 0, and find the natural frequencies in terms of k/m.

### 5. Electrical and Electronics Engineering

- Mesh analysis of a circuit gives 12I₁ − 4I₂ = 24, −4I₁ + 10I₂ − 2I₃ = 0 and −2I₂ + 8I₃ = −6. Solve for the mesh currents with Cramer's rule.

- The state-space model of an RLC circuit is ẋ = Ax with A = [[0, 1], [−1/(LC), −R/L]]. For R = 4 Ω, L = 1 H and C = 0.25 F, find the eigenvalues of A. Classify the circuit as over-, critically or under-damped, and say whether it is stable.

### 6. Food Engineering

- A juice blender mixes three concentrates to meet targets for sugar, acidity and vitamin C. This gives 0.1x + 0.2y + 0.15z = 15, 0.02x + 0.01y + 0.03z = 2 and 0.5x + 0.2y + 0.4z = 40. Decide using ranks whether the targets can be met exactly, and solve the system.

- The moisture transfer between the surface, middle and core layers of a drying grain kernel is modelled by dM/dt = AM with A = [[−2, 1, 0], [1, −2, 1], [0, 1, −1]]. Find the eigenvalues of A and explain how the smallest eigenvalue in magnitude controls the drying time.

### 7. Mechanical Engineering

- For the spring–mass system of Topic 2 (M = diag(2, 1), K = [[300, −100], [−100, 100]]), solve det(K − ω²M) = 0 to get the two natural frequencies, and find and sketch the corresponding mode shapes.

- Use the Cayley–Hamilton theorem to find A⁻¹ and A³ for the inertia-like matrix A = [[4, 1], [2, 3]]. Check A⁻¹ by direct computation.

### 8. Petroleum Engineering

- The material balance for three reservoir compartments linked by transmissibility gives 3p₁ − p₂ = 4000, −p₁ + 3p₂ − p₃ = 1000 and −p₂ + 2p₃ = 2500 (psi). Solve for the compartment pressures with Gauss–Jordan elimination.

- The permeability tensor of an anisotropic formation is k = [[200, 50], [50, 100]] mD. Find the principal permeabilities and the directions of maximum and minimum permeability, and explain how these directions affect horizontal well placement.

# Topic 4: Analytical Geometry, Coordinate Transformation, Solid Geometry, and Polar, Cylindrical and Spherical Coordinates

## Introduction

Analytical geometry describes geometric objects with algebraic equations. In the plane this means straight lines, circles and the conic sections (parabola, ellipse, hyperbola), which all come from the general second-degree equation ax² + 2hxy + by² + 2gx + 2fy + c = 0.

Coordinate transformation changes the reference frame:

- translation moves the origin: x = X + h, y = Y + k;

- rotation turns the axes through θ: x = X cos θ − Y sin θ, y = X sin θ + Y cos θ. Choosing tan 2θ = 2h/(a − b) removes the xy term, which reduces a conic to standard form.

Solid (3-D) geometry covers direction cosines and ratios, the equations of lines and planes, the angle between planes, the distance from a point to a plane, and quadric surfaces (sphere, cylinder, cone, ellipsoid, paraboloid, hyperboloid).

Curvilinear coordinate systems make problems with symmetry much simpler:

| System | Coordinates | Relation to Cartesian | Volume element |
| --- | --- | --- | --- |
| Polar (2-D) | (r, θ) | x = r cos θ, y = r sin θ | dA = r dr dθ |
| Cylindrical | (r, θ, z) | x = r cos θ, y = r sin θ, z = z | dV = r dr dθ dz |
| Spherical | (ρ, θ, φ) | x = ρ sin φ cos θ, y = ρ sin φ sin θ, z = ρ cos φ | dV = ρ² sin φ dρ dθ dφ |

Cylindrical coordinates suit pipes, shafts and wells. Spherical coordinates suit tanks, antennas and radial flow.

## Application Questions

### 1. Agricultural Engineering

- A centre-pivot irrigation system covers a circle of radius 400 m centred at (500, 300) m in the farm's coordinate grid. Write the equation of the irrigated boundary, convert it to polar coordinates centred at the pivot, and find the irrigated area.

- A grain silo is a cylinder of radius 3 m and height 10 m with a conical roof of height 2 m. Write the equations of both surfaces in cylindrical coordinates, and find the total storage volume.

### 2. Chemical Engineering

- A spherical catalyst pellet of radius R = 5 mm has a reactant concentration C(ρ) that depends only on the radial distance. Write the position of a point on the pellet surface in spherical coordinates, and explain why spherical coordinates reduce the 3-D diffusion problem to one variable.

- The cross-section of an elliptical storage tank is 9x² + 4y² − 36x + 24y + 36 = 0. Use a translation of axes to reduce it to standard form, and find the centre, semi-axes and cross-sectional area.

### 3. Computer Engineering

- A robot arm's end-effector is located by (r, θ, z) = (0.8 m, 60°, 0.5 m) in cylindrical coordinates. Convert this to Cartesian coordinates, and write a function (pseudocode) that converts between Cartesian, cylindrical and spherical coordinates in both directions.

- A lidar sensor reports a point at range ρ = 20 m, azimuth θ = 45° and elevation angle 30° (so the polar angle is φ = 60°). Convert it to Cartesian coordinates, then find the distance from the point to the ground plane z = 0 and to the plane x + y + z = 10.

### 4. Civil Engineering

- A road designer needs the equation of the plane that contains the points A(0, 0, 100), B(200, 0, 104) and C(0, 150, 97) (metres) on a graded embankment. Find the plane equation, the direction cosines of its normal, and the angle it makes with the horizontal.

- The profile of a parabolic arch bridge passes through (−30, 0), (30, 0) and (0, 15) m. Find its equation. Rotate the axes by 10° to model an inclined abutment, and write the transformed equation.

### 5. Electrical and Electronics Engineering

- The radiation pattern of a dipole antenna is proportional to sin φ in spherical coordinates. Convert the pattern surface ρ = sin φ to Cartesian form, identify it, and sketch it.

- A coaxial cable has an inner conductor of radius a = 1 mm and an outer conductor of radius b = 4 mm. Describe the dielectric region with inequalities in cylindrical coordinates, and find the cross-sectional area of the dielectric using polar coordinates.

### 6. Food Engineering

- A cylindrical can with radius 4 cm and height 11 cm is heated during sterilisation. The slowest-heating point is on the axis at mid-height. Give its cylindrical coordinates relative to the centre of the base, and find its distance from the rim edge.

- An egg is approximated by the ellipsoid x²/4 + y²/4 + z²/9 = 1 (cm). Find its volume, and use a coordinate transformation (stretching) to map it onto a unit sphere. Explain why this simplifies heat-transfer analysis.

### 7. Mechanical Engineering

- A cam profile is given in polar form by r = 40 + 10 cos θ (mm). Convert it to Cartesian form, find the maximum and minimum radius (the follower lift), and find the area enclosed by the profile.

- Two machine members lie along the lines (x − 1)/2 = (y + 1)/3 = z/6 and (x − 2)/3 = (y − 1)/(−2) = (z + 1)/6. Find the angle between them and the shortest distance between the lines.

### 8. Petroleum Engineering

- Radial flow into a vertical well is analysed in cylindrical coordinates. The drainage area is a circle of radius rₑ = 300 m around a well of radius r_w = 0.1 m. Write the flow domain in cylindrical coordinates, and find the volume of a reservoir 20 m thick with 25 % porosity.

- A directional well leaves the surface at (0, 0, 0) and reaches a target at (600, 800, −2500) m. Find the equation of the straight well path, its inclination from the vertical, and its azimuth. Express the target in spherical coordinates.

# Topic 5: Elements of Functions of Several Variables; Surface Variables

## Introduction

Most engineering quantities depend on more than one variable. Temperature changes with position and time, and pressure depends on volume and temperature. A function of several variables f(x, y, …) assigns one value to each point of its domain. The main tools are:

- Level curves and surfaces: f(x, y) = c (contours) and f(x, y, z) = c (level surfaces, such as isotherms).

- Limits and continuity in more than one dimension.

- Partial derivatives: ∂f/∂x, ∂f/∂y and higher-order derivatives. Clairaut's theorem says f_xy = f_yx.

- Total differential: df = f_x dx + f_y dy, used for error and sensitivity analysis.

- Chain rule and implicit differentiation for composite and implicitly defined functions.

- Directional derivative and gradient: D_u f = ∇f · u. The gradient points in the direction of steepest increase.

- Taylor expansion in two variables, used for linearisation.

- Extrema: stationary points classified by the second-derivative test (D = f_xx f_yy − f_xy²), and constrained optimisation with Lagrange multipliers.

Surfaces and surface variables: a surface can be written explicitly as z = f(x, y), implicitly as F(x, y, z) = 0, or parametrically as r(u, v). The parameters u and v are the surface variables. The tangent plane and normal come from ∇F or from r_u × r_v, and the element of surface area is dS = |r_u × r_v| du dv. These are needed for the surface integrals in Topic 6.

## Application Questions

### 1. Agricultural Engineering

- Crop yield (t/ha) is modelled by Y(N, W) = 0.04N + 0.05W − 0.0001N² − 0.0002W² + 0.00005NW, where N is nitrogen (kg/ha) and W is water (mm). Find the input levels that maximise yield, and confirm that the point is a maximum.

- The evapotranspiration rate E = kT^0.5 R^0.8 depends on temperature T and radiation R. Use the total differential to estimate the percentage error in E when T is measured with 2 % error and R with 3 % error.

### 2. Chemical Engineering

- For a van der Waals gas, (P + a/V²)(V − b) = RT. Find ∂P/∂V at constant T and ∂P/∂T at constant V. Verify the cyclic relation (∂P/∂V)_T(∂V/∂T)_P(∂T/∂P)_V = −1.

- A cylindrical reactor of fixed volume 10 m³ is to be built with the least surface area (and so the least material cost). Use Lagrange multipliers to find the best radius and height.

### 3. Computer Engineering

- Gradient descent minimises the loss L(w₁, w₂) = (w₁ − 3)² + 2(w₂ + 1)² + w₁w₂. Compute ∇L, write the gradient-descent update rule, and carry out 3 iterations from (0, 0) with learning rate 0.1.

- The brightness of a grayscale image is modelled as I(x, y) = 100 e^{−(x² + y²)/50}. Compute the image gradient at (3, 4), its magnitude (the edge strength) and its direction, and explain how edge-detection algorithms use this.

### 4. Civil Engineering

- The ground surface of a site is z = 50 − 0.01x² − 0.02y² (m). At the point (10, 20), find the slope in the direction of the unit vector (3/5, 4/5), the direction of steepest descent (the drainage path), and the equation of the tangent plane.

- A rectangular beam cut from a circular log of diameter d is strongest in bending when bh² is largest. Use Lagrange multipliers to find b and h subject to b² + h² = d².

### 5. Electrical and Electronics Engineering

- The power dissipated in a resistor is P = V²/R. Given V = 220 ± 2 V and R = 50 ± 0.5 Ω, use partial derivatives and the total differential to estimate the maximum absolute and relative error in P.

- The electric potential near a point charge is V(x, y, z) = k/√(x² + y² + z²). Show that V satisfies Laplace's equation ∇²V = 0 away from the origin, and find the equation of the equipotential surface through (1, 2, 2).

### 6. Food Engineering

- During drying, the moisture content is M(t, T) = M₀ e^{−k(T)t} with k(T) = 0.01 e^{0.05(T − 60)}. Find ∂M/∂t and ∂M/∂T, and use the chain rule to find dM/dt when the dryer temperature rises as T = 60 + 0.5t.

- A rectangular box for packaging cereal must hold 3000 cm³. The top and bottom cost twice as much per cm² as the sides. Use Lagrange multipliers to find the dimensions that give the cheapest box.

### 7. Mechanical Engineering

- The temperature distribution in a plate is T(x, y) = 100 − x² − 2y² (°C). A heat-seeking sensor at (2, 1) moves in the direction of fastest heating. Find that direction and the rate of temperature change in it, and find the path of the sensor.

- The surface of a turbine blade is given parametrically by r(u, v) = (u cos v, u sin v, 0.5v) for 1 ≤ u ≤ 2 and 0 ≤ v ≤ π/2 (a helicoid). Find r_u × r_v, the unit normal and the element of surface area dS.

### 8. Petroleum Engineering

- The pressure distribution around a producing well is p(x, y) = pₑ − (qμ/(4πkh)) ln[(x² + y²)/rₑ²]. Find ∇p, and show that the flow (along −∇p) is directed radially towards the well.

- The oil formation volume factor Bₒ(p, T) is approximated by Bₒ = 1.2 + 0.0001(T − 150) − 0.00002(p − 3000). Find the linear (Taylor) approximation of Bₒ about (p, T) = (3000, 150), and estimate Bₒ at (2800 psi, 160 °F).

# Topic 6: Ordinary Integrals; Double, Triple, Line and Surface Integrals; Vector Calculus

## Introduction

This topic extends integration from a single variable to curves, regions, surfaces and volumes, and introduces the calculus of vector fields.

Integrals

- Ordinary (single) integrals: ∫ f(x) dx, reviewed along with improper integrals and differentiation under the integral sign.

- Double integrals: ∬_R f(x, y) dA, evaluated as iterated integrals, sometimes after changing the order of integration or converting to polar coordinates. They give area, mass, centroids and moments of inertia of plane laminae.

- Triple integrals: ∭_V f dV, evaluated in Cartesian, cylindrical or spherical coordinates, using the Jacobian for a change of variables. They give volume, mass and centre of gravity of solids.

- Line integrals: ∫_C f ds and ∫_C F · dr along a curve C. They give work done, circulation and path-dependence. A field is conservative (path-independent) when F = ∇φ.

- Surface integrals: ∬_S f dS and ∬_S F · n dS. The second is the flux of F through S.

Vector calculus

- Derivatives and integrals of vectors: dr/dt gives velocity and acceleration along a path. Vector functions are integrated component by component.

- Gradient ∇φ of a scalar field: the direction and rate of greatest increase, normal to level surfaces.

- Divergence ∇ · F: the net outflow (source strength) per unit volume.

- Curl ∇ × F: the local rotation (circulation per unit area). A field is irrotational when ∇ × F = 0.

- Flux of a vector field: the rate at which a quantity (fluid, heat, charge) crosses a surface.

These are the building blocks of the governing equations of fluid mechanics, heat transfer, electromagnetism and elasticity.

## Application Questions

### 1. Agricultural Engineering

- The rainfall intensity over a rectangular catchment 0 ≤ x ≤ 2 km, 0 ≤ y ≤ 3 km is I(x, y) = 10 + 2x + y (mm/h). Use a double integral to find the total volumetric rainfall rate (m³/h) over the catchment and the average intensity.

- Water flows through a soil profile with Darcy velocity field q = (−2x, −2y, 4z) × 10⁻⁶ m/s. Compute ∇ · q and ∇ × q. State whether the flow is incompressible and irrotational, and find a potential function if one exists.

### 2. Chemical Engineering

- A cylindrical packed-bed reactor has radius 0.5 m and length 2 m. The catalyst density varies as ρ(r) = 800(1 − 0.3r²) kg/m³. Use a triple integral in cylindrical coordinates to find the total catalyst mass.

- The velocity in a pipe of radius R is v = v_max(1 − r²/R²) k (laminar Poiseuille flow). Compute the volumetric flow rate as the flux of v through the pipe cross-section, and show that the mean velocity is v_max/2.

### 3. Computer Engineering

- A drone flies along r(t) = (10 cos t, 10 sin t, 2t) m for 0 ≤ t ≤ 2π. Find its velocity, speed and acceleration, and the total path length. Write a short algorithm that approximates this arc length numerically with the trapezoidal rule, and compare the result with the exact value.

- A heat sink on a processor chip occupies the region 0 ≤ x ≤ 2, 0 ≤ y ≤ 2, 0 ≤ z ≤ 1 (cm). The heat generation rate is q(x, y, z) = 5xy + z (W/cm³). Use a triple integral to find the total heat generated, and write it as a nested-loop numerical approximation (a Riemann sum).

### 4. Civil Engineering

- A lamina shaped like a T-beam cross-section occupies the region between y = 0 and y = 4 for −1 ≤ x ≤ 1 (the web), plus the region 4 ≤ y ≤ 5 for −3 ≤ x ≤ 3 (the flange). Use double integrals to find the centroid ȳ and the second moment of area I_x about the centroidal axis.

- A crane lifts a load along a path in the force field F = (2xy, x² + z, y) kN. Compute the work done ∫_C F · dr from (0, 0, 0) to (1, 2, 3) along two different paths. Decide whether F is conservative, and find its potential if it is.

### 5. Electrical and Electronics Engineering

- The electric field of an infinite line charge is E = (λ/(2πε₀r)) r̂. Compute the flux of E through a closed cylinder of radius a and length L coaxial with the line. Show that it equals λL/ε₀, as Gauss's law requires.

- The magnetic field around a long straight wire is B = (μ₀I/(2πr)) θ̂. Evaluate the line integral ∮ B · dr around a circle of radius a centred on the wire, and compute ∇ × B for r ≠ 0. Comment on the results.

### 6. Food Engineering

- A hemispherical dome-shaped cake of radius 10 cm has a temperature during baking of T(ρ) = 180 − 60(1 − ρ/10) °C. Use a triple integral in spherical coordinates to find the volume-averaged temperature.

- Air flows through a rectangular dryer opening 0 ≤ x ≤ 1 m, 0 ≤ z ≤ 0.5 m with velocity field v = (0, 2 + x − z, 0) m/s. Compute the volumetric air flow rate (the flux through the opening). If the air carries 0.012 kg of water per m³, find the rate of moisture removal.

### 7. Mechanical Engineering

- Find the mass moment of inertia about the z-axis of a solid cone of height h, base radius a and uniform density ρ, using a triple integral in cylindrical coordinates.

- A fluid velocity field is v = (y, −x, 0). Compute ∇ · v and ∇ × v, interpret the flow physically (rigid-body rotation), and evaluate the circulation around the unit circle in the xy-plane.

### 8. Petroleum Engineering

- A reservoir occupies the region between the surfaces z = −2000 and z = −2000 − 0.001(x² + y²) for x² + y² ≤ 500² (m). Use a triple integral in cylindrical coordinates to find the bulk rock volume. With 20 % porosity and 75 % oil saturation, estimate the oil in place.

- The pressure field in a reservoir is p(x, y) = 3000 − 0.5x² − 0.3y² (psi), and the Darcy velocity is u = −(k/μ)∇p. Find u, and compute the flux of u across the boundary of the square −100 ≤ x, y ≤ 100 m (per unit thickness) with k/μ = 0.01.

# Topic 7: Gauss's, Green's and Stokes' Theorems and Applications; Single-Valued and Multivalued Functions

## Introduction

### The integral theorems of vector calculus

These three theorems link an integral over a region to an integral over its boundary. They generalise the Fundamental Theorem of Calculus.

| Theorem | Statement | Relates |
| --- | --- | --- |
| Green's theorem (plane) | ∮_C (P dx + Q dy) = ∬_R (∂Q/∂x − ∂P/∂y) dA | line integral ↔ double integral |
| Stokes' theorem | ∮_C F · dr = ∬_S (∇ × F) · n dS | circulation ↔ flux of the curl |
| Gauss's (divergence) theorem | ∯_S F · n dS = ∭_V ∇ · F dV | outward flux ↔ volume integral of divergence |

Applications include computing areas with line integrals (A = ½∮(x dy − y dx)), deriving the continuity equation and the heat-conduction equation, Gauss's and Ampère's laws in electromagnetism, and checking whether a field is conservative.

### Single-valued and multivalued functions

A single-valued function gives exactly one output for each input. Examples are polynomials and e^z. A multivalued function gives more than one output for some inputs:

- z^{1/2} has two values, and z^{1/n} has n values;

- log z = ln|z| + i(arg z + 2kπ) has infinitely many values;

- the inverse trigonometric functions are multivalued.

To work with them we choose a principal branch, place a branch cut (a curve across which the function jumps), and identify branch points, where going round a small circle changes the value. For example, z = 0 is a branch point of √z and of log z. Getting branches right is essential for the contour integration in Topic 8 and for physical problems with slits, cracks and flows around corners.

## Application Questions

### 1. Agricultural Engineering

- A field boundary is traced by GPS as the ellipse x = 300 cos t, y = 200 sin t (m). Use Green's theorem (A = ½∮(x dy − y dx)) to find the field area. Explain how a GPS area-measuring device can use the same formula on a list of boundary points (the shoelace formula).

- The groundwater flux under a farm is q = (x², y², z²) (m/day, scaled) in the cube 0 ≤ x, y, z ≤ 1. Check the divergence theorem by computing the net outward flux both as a surface integral and as a volume integral.

### 2. Chemical Engineering

- Apply Gauss's divergence theorem to a fixed control volume V to derive the continuity equation ∂ρ/∂t + ∇ · (ρv) = 0 from the integral mass balance.

- The complex potential of flow in a corner-shaped mixing channel involves w = z^{2/3}. Find all values of w at z = 8i, give the principal value, and state the branch point. Suggest a suitable branch cut for a channel occupying the upper half-plane.

### 3. Computer Engineering

- Polygon-area routines in computer graphics are based on Green's theorem. Derive the shoelace formula A = ½ Σ (xᵢyᵢ₊₁ − xᵢ₊₁yᵢ) from ½∮(x dy − y dx), and use it on the polygon with vertices (0, 0), (4, 0), (4, 3), (2, 5) and (0, 3). Write the algorithm in pseudocode.

- A function atan2(y, x) returns the principal argument of z = x + iy in (−π, π]. Explain why arg z is multivalued. Show what happens to the computed angle as a point travels once round the origin, and describe how "phase unwrapping" algorithms fix the resulting jump.

### 4. Civil Engineering

- Use Green's theorem to find the area and centroid of the plane region bounded by y = x² and y = 2x (in metres), which models a curved slab section. Check the answer with direct double integration.

- Seepage under a dam has flux field q = (2x − y, x + 3y, −5z) (per unit time). Check whether the field is consistent with an incompressible soil (∇ · q = 0). Use the divergence theorem to find the net outflow from the unit cube.

### 5. Electrical and Electronics Engineering

- Starting from Ampère's circuital law ∮_C H · dr = I_enc, use Stokes' theorem to derive the differential form ∇ × H = J. Apply it to find H inside a long straight conductor of radius a carrying a uniformly distributed current I.

- The phase of an AC signal is found from the complex logarithm of the phasor. Find all values of log(−1 + i√3), give the principal value, and explain what the choice of branch means when measuring phase angles above 180°.

### 6. Food Engineering

- A food can is heated in a retort. Heat flux into the can is q = −k∇T. Use the divergence theorem on the can volume to show that the total rate of heat entering through the surface equals ∭ ρc_p (∂T/∂t) dV (when there is no internal generation). This is the basis of the heat-conduction equation.

- The stirring flow in a mixing bowl is approximated by v = (−y, x, 0) (rad/s units). Use Stokes' theorem to compute the circulation around the bowl rim (a circle of radius 0.15 m in the plane z = 0.1 m) through the flat disc it bounds. Check the answer against a direct line integral.

### 7. Mechanical Engineering

- Compute the work done by the force F = (y + z, z + x, x + y) round the triangle with vertices (1, 0, 0), (0, 1, 0) and (0, 0, 1) using Stokes' theorem. Explain the result in terms of conservative forces.

- The flow round a sharp re-entrant corner of angle 3π/2 in a duct is modelled by w = z^{2/3}. Identify the branch point, describe the branch cut, and find the two other values of w at z = 1 besides the principal value. Explain why the velocity becomes unbounded at the corner.

### 8. Petroleum Engineering

- Use the divergence theorem to show that for steady incompressible flow in a reservoir with no wells (∇ · u = 0), the net volumetric flux through any closed surface is zero. Extend the argument to a region containing a producing well of rate q, and derive the resulting flux condition.

- The complex potential for flow towards a well at the origin is w = (q/(2π)) log z. Show that the stream function ψ = Im w is multivalued, find its change on going once round the well, and relate this jump to the well's production rate.

# Topic 8: Analytic Functions, Cauchy–Riemann Equations, Singularities and Zeros, Contour Integration and Bilinear Transformation

## Introduction

Complex analysis studies functions f(z) = u(x, y) + i v(x, y) of the complex variable z = x + iy.

- Analytic (holomorphic) functions: f is analytic at z₀ if it is differentiable in a neighbourhood of z₀. Analytic functions are very well behaved. They are infinitely differentiable and can be expanded in power series.

- Cauchy–Riemann equations: a necessary condition for analyticity is u_x = v_y and u_y = −v_x. If the partial derivatives are also continuous, the condition is sufficient too. As a result, u and v are harmonic (∇²u = ∇²v = 0), and v is the harmonic conjugate of u. This is why complex analysis can model 2-D potential flow, electrostatics and steady heat conduction.

- Zeros and singularities: z₀ is a zero of order m if f(z₀) = … = f^{(m−1)}(z₀) = 0 but f^{(m)}(z₀) ≠ 0. Singularities are points where f is not analytic. They are classified, using the Laurent series, as removable, poles (of order m) or essential.

- Contour integration:

  - Cauchy's integral theorem: ∮_C f(z) dz = 0 if f is analytic on and inside the closed curve C.

  - Cauchy's integral formula: f(z₀) = (1/2πi)∮_C f(z)/(z − z₀) dz, and its extension to derivatives.

  - Residue theorem: ∮_C f(z) dz = 2πi Σ Res(f, zₖ). This is used to evaluate real integrals and inverse Laplace transforms.

- Bilinear (Möbius) transformation: w = (az + b)/(cz + d) with ad − bc ≠ 0. It is conformal (it preserves angles), maps circles and lines to circles and lines, and is fixed uniquely by three point correspondences. It is used to map awkward domains onto simple ones, and underlies the Smith chart.

Complex analysis is essential in control-system stability (Nyquist criterion), AC circuit analysis, signal processing (the z-transform), aerodynamics (the Joukowski aerofoil) and potential-flow problems.

## Application Questions

### 1. Agricultural Engineering

- Steady groundwater flow under a drainage field is modelled by the potential φ(x, y) = x² − y². Check that φ is harmonic. Use the Cauchy–Riemann equations to find the stream function ψ and the complex potential w = φ + iψ, and sketch the flow lines. Where is the stagnation point?

- The transfer function of a greenhouse temperature controller is G(s) = 10/((s + 1)(s + 2)(s + 5)). Find its poles and zeros, and use the residue theorem to find the impulse response g(t), the inverse Laplace transform, as (1/2πi)∮ e^{st}G(s) ds.

### 2. Chemical Engineering

- Use the Cauchy–Riemann equations to show that f(z) = z̄ = x − iy is not analytic anywhere, while f(z) = e^{x}(cos y + i sin y) is analytic everywhere. Identify the second function as e^z. Explain why analyticity is needed when the concentration field in a thin 2-D film is modelled as the real part of an analytic function.

- The response of a first-order-plus-lag process is C(s) = 1/(s(τs + 1)²). Classify its singularities, and use residues to find c(t) for τ = 2 min.

### 3. Computer Engineering

- A digital filter has transfer function H(z) = (z² − 1)/(z² − 0.5z + 0.06). Find its zeros and poles, and state whether the filter is stable (all poles inside |z| = 1). Write a program sketch that computes the poles and tests stability for any given denominator coefficients.

- The inverse z-transform is x[n] = (1/2πi)∮ X(z) z^{n−1} dz. Use the residue theorem to find x[n] for X(z) = z/((z − 0.5)(z − 0.25)) with |z| > 0.5.

### 4. Civil Engineering

- Seepage under a sheet pile is modelled by the complex potential w = cosh⁻¹(z/b). Rewrite it as z = b cosh w, separate the real and imaginary parts, and show that the equipotential lines are ellipses and the streamlines are hyperbolae. Identify the branch points at z = ±b.

- Evaluate the real integral ∫_{−∞}^{∞} dx/(x² + 4)² by contour integration over a semicircle in the upper half-plane. The integral appears in the deflection of a beam on an elastic foundation under a distributed load.

### 5. Electrical and Electronics Engineering

- Find the bilinear transformation that maps z = 0, 1, ∞ to w = −1, 0, 1. Show that the normalised load-impedance plane (Re z ≥ 0) is mapped onto the unit disc |w| ≤ 1, and explain how this gives the Smith chart reflection coefficient Γ = (z − 1)/(z + 1).

- An RLC circuit has transfer function H(s) = 1/(s² + 2s + 5). Find its poles, and classify them as simple or of higher order. Use Cauchy's integral formula / residues to find the impulse response h(t).

### 6. Food Engineering

- The temperature in a long rectangular food slab in steady state is T(x, y) = 20 + 5(x³ − 3xy²). Show that T is harmonic, and find the harmonic conjugate that gives the heat-flow lines. Write the analytic function f(z) whose real part is T − 20.

- Evaluate ∮_C (e^{2z}/(z − 1)³) dz where C is |z| = 2, using Cauchy's integral formula for derivatives. Such integrals arise in inverting Laplace-domain models of thermal processing with repeated time constants.

### 7. Mechanical Engineering

- The Joukowski transformation w = z + 1/z maps circles in the z-plane to aerofoil-like shapes. Find the singularities and the critical points (where dw/dz = 0) of the transformation, and show that the circle |z| = 1 maps to the segment [−2, 2] on the real axis.

- Evaluate ∫₀^{2π} dθ/(5 + 4 cos θ) by contour integration round the unit circle. This type of integral appears in the analysis of rotating machinery with eccentric loading.

### 8. Petroleum Engineering

- A source (injector) at z = −a and a sink (producer) at z = a give the complex potential w = (q/2π)[log(z + a) − log(z − a)]. Find the singularities of w and their type, and show that the equipotentials are circles (Apollonius circles). Find the stagnation points, if there are any.

- Find the bilinear transformation that maps the circle |z| = R (the reservoir boundary) to the unit circle, with an off-centre well at z = z₀ going to the centre w = 0. Explain how this lets the known solution for a centred well be used for an off-centre well.