GET 321

Engineering Mathematics IV

Marking Scheme for the Theory Questions

Questions 2 to 7  ·  Types 1 to 5

2024/2025 & 2025/2026 Second Semester B. Eng. Examination

September 2026

Contents

Right-click here and choose Update Field to show the table of contents.

# Type 1

## Question 2: Differential Equations and Free Vibration

### (a) Non-homogeneous second-order ODE

Question 2(a). Find the solution to the engineering problem represented by the differential equation below. (6 marks)

d2ydx2−4dydx+4y=x2+x3e2x+sin2x

Complementary function. The auxiliary equation is

m2−4m+4=(m−2)2=0 ⇒ m=2 (repeated)

yc=(A+Bx)e2x

Particular integral. Treat each term on the right separately: yp=y1+y2+y3.

For x2. Try y1=ax2+bx+c, so y1′=2ax+b and y1″=2a:

2a−4(2ax+b)+4(ax2+bx+c)=4ax2+(4b−8a)x+(2a−4b+4c)=x2

4a=1, 4b−8a=0, 2a−4b+4c=0 ⇒ a=14, b=12, c=38

For x3e2x. Here e2x is part of yc (twice over), so put y2=u(x)e2x. Then y2′=(u′+2u)e2x, y2″=(u″+4u′+4u)e2x and

y2″−4y2′+4y2=u″+4u′+4u−4u′−8u+4ue2x=u″e2x

So u″=x3, giving u=x520 and y2=x520e2x.

For sin2x. Try y3=Ccos2x+Dsin2x, so y3″=−4y3:

−4y3−4y3′+4y3=−4(−2Csin2x+2Dcos2x)=8Csin2x−8Dcos2x=sin2x

so C=18, D=0 and y3=18cos2x.

Answer. y=(A+Bx)e2x+x520e2x+x24+x2+38+18cos2x

### (b) Undamped spring–mass system

Question 2(b). A spring with a mass of 2000 g has natural length of one-half a meter. A force of 25.6 N is required to maintain it stretched to a length of 700 mm. If the spring is stretched to a length of 700 mm and then released with initial velocity 0 m/s: (i) What will be the position of the mass at any time, t? (ii) What length of travel (in m) would the mass achieve in t=π/24 seconds? (6 marks)

Spring constant. Hooke’s law, F=kx, with the extension x=0.7−0.5=0.2 m:

k(0.2)=25.6 ⇒ k=128 N/m

The mass is m=2000 g=2 kg. Let x(t) be the displacement of the mass from equilibrium (the natural length, 0.5 m), positive in the direction of stretching.

Equation of motion. There is no damping, so

md2xdt2+kx=0 ⇒ 2d2xdt2+128x=0 ⇒ d2xdt2+64x=0

The auxiliary equation r2+64=0 gives r=±8i, so

x(t)=c1cos8t+c2sin8t

Initial conditions. The spring starts stretched to 0.7 m, so x(0)=0.2, and it is released from rest, so x′(0)=0.

x(0)=c1=0.2,  x′(0)=8c2=0 ⇒ c2=0

(i) Position at any time.

x(t)=0.2cos8t m

The mass oscillates about the equilibrium position with amplitude 0.2 m, angular frequency ω=8 rad/s and period T=2π/8=π/4 s.

(ii) At t=π/24 s.

xπ24=0.2cos8π24=0.2cosπ3=0.2×0.5=0.1 m

The velocity x′(t)=−1.6sin8t stays negative for 0<t<π/8, and π/24<π/8, so the mass moves steadily back towards equilibrium without turning. The distance travelled from the start is therefore

0.2−0.1=0.1 m

and the spring’s length at that moment is 0.5+0.1=0.6 m.

Answer. (i) x(t)=0.2cos8t m. (ii) At t=π/24 s the mass is 0.1 m from equilibrium, having travelled 0.1 m from its starting point (spring length 0.6 m).

## Question 3: RLC Circuit

Question 3. A series circuit comprises a resistor of value 20 Ω with a 1 H inductor and a capacitor whose capacitance is 2000 μF. If the initial charge and current are both zero, find the charge at time t in these two cases: (i) the circuit is connected to a 12-volt battery; (ii) the circuit is connected to a generator producing a voltage of E(t)=12sin10t. (6 marks) Find also the current in each case in (i) and (ii). (6 marks)

The circuit equation. By Kirchhoff’s voltage law, with I=dq/dt,

Ld2qdt2+Rdqdt+qC=E(t)

Here R=20 Ω, L=1 H and C=2000 μF=2×10−3 F, so 1/C=500:

d2qdt2+20dqdt+500q=E(t),  q(0)=0, I(0)=q′(0)=0

Complementary function. r2+20r+500=0 gives r=−10±100−500=−10±20i:

qc=e−10tc1cos20t+c2sin20t

### Case (i): 12 V battery, E=12

Particular integral. Try a constant: 500qp=12, so qp=12500=0.024.

q=e−10tc1cos20t+c2sin20t+0.024

q(0)=0:  c1+0.024=0⇒c1=−0.024.

q′(0)=0:  −10c1+20c2=0⇒c2=12c1=−0.012.

q(t)=0.024−e−10t0.024cos20t+0.012sin20t=32502−e−10t2cos20t+sin20t

Current. Differentiating,

I=dqdt=10e−10t0.024cos20t+0.012sin20t−e−10t−0.48sin20t+0.24cos20t=0.6 e−10tsin20t

### Case (ii): generator, E=12sin10t

Particular integral. Try qp=Acos10t+Bsin10t, so qp″=−100qp:

(500−100)(Acos10t+Bsin10t)+20(−10Asin10t+10Bcos10t)=12sin10t

cos10t: 400A+200B=0,  sin10t: 400B−200A=12

From the first, B=−2A; then −800A−200A=12, so A=−0.012 and B=0.024:

qp=−0.012cos10t+0.024sin10t

Constants. q(0)=c1−0.012=0⇒c1=0.012.

q′(0)=−10c1+20c2+10(0.024)=−0.12+20c2+0.24=0⇒c2=−0.006.

q(t)=e−10t0.012cos20t−0.006sin20t−0.012cos10t+0.024sin10t

Current.

I=dqdt=−e−10t0.24cos20t+0.18sin20t+0.24cos10t+0.12sin10t

Check: I(0)=−0.24+0.24=0. ✓ The e−10t terms are the transient, which dies away; the steady-state current is 0.24cos10t+0.12sin10t.

Answer. (i) q=0.024−e−10t(0.024cos20t+0.012sin20t) C and I=0.6 e−10tsin20t A. (ii) q=e−10t(0.012cos20t−0.006sin20t)−0.012cos10t+0.024sin10t C and I=−e−10t(0.24cos20t+0.18sin20t)+0.24cos10t+0.12sin10t A.

## Question 4: Series Solutions and Bessel’s Equation

### (a) Power series solution

Question 4(a). Find the power series solution of y″+y=0. (6 marks)

Assume a power series solution about x=0:

y=n=0∞anxn,  y″=n=2∞n(n−1)anxn−2=n=0∞(n+2)(n+1)an+2 xn

Substituting into y″+y=0:

n=0∞(n+2)(n+1)an+2+anxn=0

Every coefficient must vanish, which gives the recurrence relation

an+2=−an(n+2)(n+1),  n=0,1,2,…

Even coefficients (in terms of a0):

a2=−a02!,  a4=−a24⋅3=a04!,  a6=−a46⋅5=−a06!, …

Odd coefficients (in terms of a1):

a3=−a13!,  a5=−a35⋅4=a15!,  a7=−a17!, …

Hence

y=a01−x22!+x44!−x66!+⋯+a1x−x33!+x55!−x77!+⋯

The two series are the Maclaurin series of cosx and sinx, and both converge for all x.

Answer. y=a01−x22!+x44!−⋯+a1x−x33!+x55!−⋯=a0cosx+a1sinx, where a0 and a1 are arbitrary constants.

### (b) Bessel’s equation

Question 4(b). Find the general solution, in terms of Bessel functions, of the equation  xy″+y′+12y=0. (6 marks)

The fraction in the equation reduces to 12, so the equation is

xy″+y′+12 y=0

Change of variable. Equations of the form xy″+y′+λy=0 become Bessel’s equation under z=2λx. Here λ=12, so put

z=2x2=2x,  z2=2x,  dzdx=1z

Then

dydx=1zdydz,  d2ydx2=1zddz1zdydz=1z2d2ydz2−1z3dydz

With x=z2/2:

xd2ydx2+dydx+12y=12d2ydz2−12zdydz+1zdydz+12y=12d2ydz2+1zdydz+y=0

Multiplying by 2z2:

z2d2ydz2+zdydz+(z2−ν2)y=0 with ν=0

This is Bessel’s equation of order ν=0.

General solution. For order ν=0 the two independent solutions are J0(z) and the Bessel function of the second kind, Y0(z):

y=c1J0(z)+c2Y0(z)=c1J0​2x+c2Y0​2x

In series form, the first solution is

J0​2x=m=0∞(−1)m(m!)2x2m=1−x2+x216−x3288+⋯

Answer. y=c1J0​2x+c2Y0​2x.

## Question 5: Eigenvalues and Eigenvectors

### (a) Eigenvalues and eigenvectors of a 3 × 3 matrix

Question 5(a). Find the eigenvalues and eigenvectors of the matrix (6 marks)

A=314026005

Eigenvalues. A is upper triangular, so det(A−λI) is the product of the diagonal entries:

det(A−λI)=(3−λ)(2−λ)(5−λ)=0 ⇒ λ=2, 3, 5

(Expanded, the characteristic equation is λ3−10λ2+31λ−30=0.)

Eigenvectors. Solve (A−λI)x=0 for each eigenvalue.

λ=2:  114006003x=0 ⇒ x3=0, x1+x2=0 ⇒ x=1−10

λ=3:  0140−16002x=0 ⇒ x3=0, x2=0, x1 free ⇒ x=100

λ=5:  −2140−36000x=0 ⇒ x2=2x3, −2x1+2x3+4x3=0⇒x1=3x3 ⇒ x=321

Check for λ=5: A321=9+2+44+65=15105=5321 ✓

Answer. λ1=2 with x1=k(1,−1,0)T;  λ2=3 with x2=k(1,0,0)T;  λ3=5 with x3=k(3,2,1)T (k≠0).

### (b) Principal directions of a stretched membrane

Question 5(b). An elastic membrane in the x1x2-plane with boundary circle x12+x22=1 is stretched so that a point P:(x1,x2) goes over into the point Q:(y1,y2) given by

y=y1y2=Ax=5335x1x2

Find the principal directions, that is, the directions of the position vector x of P for which the direction of the position vector y of Q is the same or exactly opposite. (6 marks)

We need the vectors x for which y=Ax is parallel to x, that is Ax=λx. These are the eigenvectors of

A=5335

Eigenvalues.

det(A−λI)=(5−λ)2−9=λ2−10λ+16=(λ−8)(λ−2)=0 ⇒ λ1=8, λ2=2

Eigenvectors.

For λ1=8:  −3x1+3x2=0 ⇒ x2=x1, so x(1)=11, at 45∘ to the x1-axis.

For λ2=2:  3x1+3x2=0 ⇒ x2=−x1, so x(2)=1−1, at 135∘ (or −45∘) to the x1-axis.

Both eigenvalues are positive, so in each principal direction y points the same way as x. The two directions are perpendicular because A is symmetric.

Interpretation. Along the 45∘ direction the membrane is stretched by a factor of 8, and along the 135∘ direction by a factor of 2. With coordinates u1,u2 along these principal directions, the boundary circle x12+x22=1 becomes the ellipse

u1282+u2222=1

Answer. The principal directions are x2=x1 (at 45∘, eigenvalue 8, stretch factor 8) and x2=−x1 (at 135∘, eigenvalue 2, stretch factor 2). The circle is deformed into an ellipse with semi-axes 8 and 2 along these directions.

## Question 6: Legendre’s Equation and the Wave Equation

### (a) Legendre’s equation

Question 6(a). Solve this special Legendre equation, (1−x2)y″−2xy′+2y=0, which occurs in models exhibiting spherical symmetry. (6 marks)

This is Legendre’s equation (1−x2)y″−2xy′+n(n+1)y=0 with n(n+1)=2, that is n=1.

Power series. Put y=m=0∞amxm. Then

(1−x2)m​m(m−1)amxm−2−2xm​mamxm−1+2m​amxm=0

Collecting the coefficient of xm:

(m+2)(m+1)am+2−m(m−1)+2m−2am=0

Since m(m−1)+2m−2=m2+m−2=(m+2)(m−1), the recurrence relation is

am+2=(m+2)(m−1)(m+2)(m+1) am=m−1m+1 am

Odd series (m=1): a3=0⋅a1=0, so a5=a7=⋯=0 and the series stops:

y1=a1x=a1P1(x)

Even series (from a0):

a2=−a0,  a4=13a2=−a03,  a6=35a4=−a05,  a8=−a07, …

y2=a01−x2−x43−x65−x87−⋯=a01−x2ln1+x1−x,  |x|<1

because 12ln1+x1−x=x+x33+x55+⋯.

Check of y1=x:  (1−x2)(0)−2x(1)+2x=0. ✓

Answer. y=c1x+c21−x2−x43−x65−⋯=c1P1(x)+c21−x2ln1+x1−x for x<1. The polynomial solution is the Legendre polynomial P1(x)=x.

### (b) One-dimensional wave equation

Question 6(b). Find the solution to this one-dimensional wave equation:  ∂2u∂x2=116∂2u∂t2 for 0<x<2, t>0. The boundary conditions are u(0,t)=u(2,t)=0. The initial conditions are (i) u(x,0)=6sinπx−3sin4πx and (ii) ∂u∂t(x,0)=0. (6 marks)

The equation is

∂2u∂x2=116∂2u∂t2 ⇔ ∂2u∂t2=c2∂2u∂x2,  c2=16, c=4, L=2

Separation of variables. Put u=F(x)G(t):

F″F=G″16 G=−p2

so F″+p2F=0 and G″+16p2G=0.

Boundary conditions. u(0,t)=u(2,t)=0 need F(0)=F(2)=0. With F=Acospx+Bsinpx, F(0)=0 gives A=0, and sin2p=0 gives

p=nπ2,  Fn(x)=sinnπx2,  n=1,2,3,…

Time part. Gn″+λn2Gn=0 with λn=cnπL=4nπ2=2nπ:

Gn(t)=Bncos2nπt+Bn*sin2nπt

so

u(x,t)=n=1∞Bncos2nπt+Bn*sin2nπtsinnπx2

Initial velocity. ∂u∂t(x,0)=∑2nπBn*sinnπx2=0, so Bn*=0 for every n.

Initial displacement.

u(x,0)=n=1∞Bnsinnπx2=6sinπx−3sin4πx

sinπx=sin2πx2 is the n=2 term and sin4πx=sin8πx2 is the n=8 term. Comparing coefficients (no Fourier integrals are needed):

B2=6,  B8=−3,  Bn=0 otherwise

With λ2=4π and λ8=16π:

Answer. u(x,t)=6sinπxcos4πt−3sin4πxcos16πt.

Check. u(0,t)=u(2,t)=0 since sin2π=sin8π=0; u(x,0)=6sinπx−3sin4πx; ut(x,0)=0; and each term satisfies utt=16uxx, for example (4π)2=16π2. ✓

## Question 7: Numerical Differentiation

### (a) Rate of population growth

Question 7(a). The table below gives the census population (in millions) of a state for the years 1961 to 2001. Find the rate of growth of the population in the year 2001. (6 marks)

| Year (x) | 1961 | 1971 | 1981 | 1991 | 2001 |
| --- | --- | --- | --- | --- | --- |
| Population (y) | 19.96 | 36.65 | 58.81 | 77.21 | 94.61 |

The year 2001 is at the end of the table, so we use Newton’s backward difference formula with h=10 years.

Backward difference table.

| Year x | y | ∇y | ∇2y | ∇3y | ∇4y |
| --- | --- | --- | --- | --- | --- |
| 1961 | 19.96 |  |  |  |  |
| 1971 | 36.65 | 16.69 |  |  |  |
| 1981 | 58.81 | 22.16 | 5.47 |  |  |
| 1991 | 77.21 | 18.40 | −3.76 | −9.23 |  |
| 2001 | 94.61 | 17.40 | −1.00 | 2.76 | 11.99 |

Derivative at the last point xn=2001 (where p=0):

dydxxn=1h∇yn+12∇2yn+13∇3yn+14∇4yn

=11017.40+−1.002+2.763+11.994=11017.40−0.50+0.92+2.9975=20.817510=2.0818

Answer. In 2001 the population was growing at about 2.08 million per year.

### (b) Derivatives from tabulated data

Question 7(b). Given the following data points, determine dydx and d2ydx2 at x=1.1 and x=1.5. (6 marks)

| x | 1.0 | 1.1 | 1.2 | 1.3 | 1.4 | 1.5 | 1.6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| f(x) | 7.989 | 8.403 | 8.781 | 9.129 | 9.451 | 9.750 | 10.031 |

Here h=0.1. The point x=1.1 is near the start of the table, so we use Newton’s forward formula there; x=1.5 is near the end, so we use the backward formula.

Difference table (row xi lists Δkfi, the forward differences starting at that row).

| x | f(x) | Δ | Δ2 | Δ3 | Δ4 | Δ5 | Δ6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.0 | 7.989 | 0.414 | −0.036 | 0.006 | −0.002 | 0.001 | 0.002 |
| 1.1 | 8.403 | 0.378 | −0.030 | 0.004 | −0.001 | 0.003 |  |
| 1.2 | 8.781 | 0.348 | −0.026 | 0.003 | 0.002 |  |  |
| 1.3 | 9.129 | 0.322 | −0.023 | 0.005 |  |  |  |
| 1.4 | 9.451 | 0.299 | −0.018 |  |  |  |  |
| 1.5 | 9.750 | 0.281 |  |  |  |  |  |
| 1.6 | 10.031 |  |  |  |  |  |  |

### At x=1.1 (forward differences, x0=1.1)

From the table, Δy0=0.378, Δ2y0=−0.030, Δ3y0=0.004, Δ4y0=−0.001 and Δ5y0=0.003.

dydxx0=1hΔy0−12Δ2y0+13Δ3y0−14Δ4y0+15Δ5y0

=10.10.378+0.0302+0.0043+0.0014+0.0035=100.378+0.015+0.00133+0.00025+0.0006=3.952

d2ydx2x0=1h2Δ2y0−Δ3y0+1112Δ4y0−56Δ5y0

=100−0.030−0.004+1112(−0.001)−56(0.003)=100−0.030−0.004−0.00092−0.0025=−3.742

### At x=1.5 (backward differences, xn=1.5)

From the table, ∇yn=0.299, ∇2yn=−0.023, ∇3yn=0.003, ∇4yn=−0.001 and ∇5yn=0.001.

dydxxn=1h∇yn+12∇2yn+13∇3yn+14∇4yn+15∇5yn

=100.299−0.0115+0.001−0.00025+0.0002=2.885

d2ydx2xn=1h2∇2yn+∇3yn+1112∇4yn+56∇5yn

=100−0.023+0.003−0.00092+0.00083=−2.008

Answer. At x=1.1: dydx≈3.952 and d2ydx2≈−3.742. At x=1.5: dydx≈2.885 and d2ydx2≈−2.008.

# Type 2

## Question 2: Differential Equations and Damped Vibration

### (a) Non-homogeneous second-order ODE

Question 2(a). An engineering problem is represented by the differential equation below. Give the solution to the problem. (6 marks)

d2ydx2+dydx+y=x2+xe2x+excos2x

Complementary function. The auxiliary equation is

m2+m+1=0 ⇒ m=−1±1−42=−12±32i

yc=e−x/2Acos32x+Bsin32x

Particular integral. yp=y1+y2+y3, one part for each term on the right. None of the terms appears in yc.

For x2. Try y1=ax2+bx+c:

2a+(2ax+b)+(ax2+bx+c)=ax2+(2a+b)x+(2a+b+c)=x2

a=1, 2a+b=0⇒b=−2, 2a+b+c=0⇒c=0 ⇒ y1=x2−2x

For xe2x. Put y2=u e2x. Then y2′=(u′+2u)e2x, y2″=(u″+4u′+4u)e2x and

y2″+y2′+y2=u″+5u′+7ue2x=xe2x

Try u=px+q:  5p+7(px+q)=x⇒p=17, q=−549, so

y2=7x−549e2x

For excos2x. Put y3=v ex. Then y3′=(v′+v)ex, y3″=(v″+2v′+v)ex and

y3″+y3′+y3=v″+3v′+3vex=excos2x

Try v=Ccos2x+Dsin2x, with v″=−4v:

(−C+6D)cos2x+(−D−6C)sin2x=cos2x ⇒ −C+6D=1, D=−6C

so −37C=1, C=−137, D=637, and

y3=ex376sin2x−cos2x

Answer. y=e−x/2Acos32x+Bsin32x+x2−2x+7x−549e2x+ex376sin2x−cos2x

### (b) Damped spring–mass system

Question 2(b). A spring with a 2000-g mass has a natural length 0.5 m, and a 25.6-newton force is required to maintain it stretched to a length of 0.7 m. If the spring is immersed in a fluid with damping constant β=40 and the mass is released from the equilibrium position with initial velocity 0.6 m/s: (i) characterize the damped spring–mass system with reason(s); (ii) find the position of the mass at any time t. (6 marks)

Data. m=2 kg. From Hooke’s law, k(0.7−0.5)=25.6, so k=128 N/m. The damping constant is β=40.

Equation of motion.

md2xdt2+βdxdt+kx=0 ⇒ 2x″+40x′+128x=0 ⇒ x″+20x′+64x=0

(i) Characterisation. The type of motion depends on the sign of β2−4mk:

β2−4mk=402−4(2)(128)=1600−1024=576>0

The auxiliary equation has two distinct real roots,

r2+20r+64=(r+4)(r+16)=0 ⇒ r=−4, −16

so the system is overdamped: the damping is strong enough that the mass does not oscillate, and it returns to equilibrium without passing through it more than once.

(ii) Position.

x(t)=c1e−4t+c2e−16t

The mass starts at equilibrium with velocity 0.6 m/s, so x(0)=0 and x′(0)=0.6.

c1+c2=0,  −4c1−16c2=0.6 ⇒ 12c1=0.6, c1=0.05, c2=−0.05

Answer. (i) Overdamped, because β2−4mk=576>0 (real roots −4 and −16, no oscillation). (ii) x(t)=0.05e−4t−e−16t m.

## Question 3: RLC Circuit

Question 3. Find (a) the charge (6 marks) and (b) the current (6 marks) at time t in a circuit with R=40 Ω, L=1 H, C=16×10−4 F and E(t)=50cos5t. The initial charge and current are both 0.

The circuit equation. With 1/C=1/(16×10−4)=625,

Ld2qdt2+Rdqdt+qC=E(t) ⇒ d2qdt2+40dqdt+625q=50cos5t

with q(0)=0 and q′(0)=I(0)=0.

Complementary function. r2+40r+625=0 gives r=−20±400−625=−20±15i:

qc=e−20tc1cos15t+c2sin15t

Particular integral. Try qp=Acos5t+Bsin5t, with qp″=−25qp:

(625−25)(Acos5t+Bsin5t)+40(−5Asin5t+5Bcos5t)=50cos5t

cos5t: 600A+200B=50,  sin5t: 600B−200A=0

The second gives A=3B; then 1800B+200B=50, so B=140 and A=340:

qp=1403cos5t+sin5t

Constants. q(0)=c1+340=0⇒c1=−340.

q′(0)=−20c1+15c2+5⋅140=32+15c2+18=0⇒c2=−13120.

(a) Charge.

q(t)=−e−20t340cos15t+13120sin15t+1403cos5t+sin5t

(b) Current. Differentiating,

I(t)=dqdt=e−20t7924sin15t−18cos15t+18cos5t−3sin5t

Check: I(0)=−18+18=0. ✓

Answer. (a) q=−e−20t0.075cos15t+0.1083sin15t+0.075cos5t+0.025sin5t C. (b) I=e−20t3.2917sin15t−0.125cos15t+0.125cos5t−0.375sin5t A. The e−20t terms are the transient; the steady-state current is 18(cos5t−3sin5t).

## Question 4: Series Solutions and Bessel’s Equation

### (a) Power series solution

Question 4(a). Find the power series solution of 2(y″+2y)=0. (6 marks)

Dividing by 2, the equation is y″+2y=0. Assume y=n=0∞anxn, so y″=n=0∞(n+2)(n+1)an+2xn. Substituting:

n=0∞(n+2)(n+1)an+2+2anxn=0 ⇒ an+2=−2an(n+2)(n+1)

Even coefficients:

a2=−2a02!,  a4=−2a24⋅3=22a04!,  a6=−23a06!, …

Odd coefficients:

a3=−2a13!,  a5=22a15!,  a7=−23a17!, …

Hence

y=a01−2x22!+4x44!−8x66!+⋯+a1x−2x33!+4x55!−8x77!+⋯

Since 2kx2k=(2x)2k, the first series is cos2x and the second is 12sin2x.

Answer. y=a01−x2+x46−⋯+a1x−x33+x530−⋯=a0cos2x+a12sin2x

### (b) Bessel’s equation

Question 4(b). Find the general solution, in terms of Bessel functions, of the equation  xy″+y′+24y=0. (6 marks)

The fraction in the equation reduces to 12, so the equation is

xy″+y′+12 y=0

Change of variable. Equations of the form xy″+y′+λy=0 become Bessel’s equation under z=2λx. Here λ=12, so put

z=2x2=2x,  z2=2x,  dzdx=1z

Then

dydx=1zdydz,  d2ydx2=1zddz1zdydz=1z2d2ydz2−1z3dydz

With x=z2/2:

xd2ydx2+dydx+12y=12d2ydz2−12zdydz+1zdydz+12y=12d2ydz2+1zdydz+y=0

Multiplying by 2z2:

z2d2ydz2+zdydz+(z2−ν2)y=0 with ν=0

This is Bessel’s equation of order ν=0.

General solution. For order ν=0 the two independent solutions are J0(z) and the Bessel function of the second kind, Y0(z):

y=c1J0(z)+c2Y0(z)=c1J0​2x+c2Y0​2x

In series form, the first solution is

J0​2x=m=0∞(−1)m(m!)2x2m=1−x2+x216−x3288+⋯

Answer. y=c1J0​2x+c2Y0​2x.

## Question 5: Eigenvalues and Eigenvectors

### (a) Eigenvalues and eigenvectors of a 3 × 3 matrix

Question 5(a). Find the eigenvalues and eigenvectors of the matrix (6 marks)

A=1−10−12−10−11

Characteristic equation. Expanding along the first row,

det(A−λI)=(1−λ)(2−λ)(1−λ)−1+1⋅−(1−λ)=(1−λ)(2−λ)(1−λ)−2

=(1−λ)(λ2−3λ)=−λ(λ−1)(λ−3)=0

so λ=0, 1, 3. (Expanded: λ3−4λ2+3λ=0.)

Eigenvectors.

λ=0:  x1−x2=0 and −x2+x3=0 ⇒ x=(1,1,1)T

λ=1:  0−10−11−10−10x=0 ⇒ x2=0, x1=−x3 ⇒ x=(1,0,−1)T

λ=3:  −2−10−1−1−10−1−2x=0 ⇒ x2=−2x1, x2=−2x3 ⇒ x=(1,−2,1)T

Check for λ=3: A(1,−2,1)T=(1+2, −1−4−1, 2+1)T=(3,−6,3)T=3(1,−2,1)T ✓. The matrix is symmetric, so the three eigenvectors are mutually orthogonal.

Answer. λ1=0 with x1=k(1,1,1)T;  λ2=1 with x2=k(1,0,−1)T;  λ3=3 with x3=k(1,−2,1)T (k≠0).

### (b) Principal directions of a stretched membrane

Question 5(b). An elastic membrane in the x1x2-plane with boundary circle x12+x22=1 is stretched so that a point P:(x1,x2) goes over into the point Q:(y1,y2) given by

y=y1y2=Ax=5335x1x2

Find the principal directions, that is, the directions of the position vector x of P for which the direction of the position vector y of Q is the same or exactly opposite. (6 marks)

We need the vectors x for which y=Ax is parallel to x, that is Ax=λx. These are the eigenvectors of

A=5335

Eigenvalues.

det(A−λI)=(5−λ)2−9=λ2−10λ+16=(λ−8)(λ−2)=0 ⇒ λ1=8, λ2=2

Eigenvectors.

For λ1=8:  −3x1+3x2=0 ⇒ x2=x1, so x(1)=11, at 45∘ to the x1-axis.

For λ2=2:  3x1+3x2=0 ⇒ x2=−x1, so x(2)=1−1, at 135∘ (or −45∘) to the x1-axis.

Both eigenvalues are positive, so in each principal direction y points the same way as x. The two directions are perpendicular because A is symmetric.

Interpretation. Along the 45∘ direction the membrane is stretched by a factor of 8, and along the 135∘ direction by a factor of 2. With coordinates u1,u2 along these principal directions, the boundary circle x12+x22=1 becomes the ellipse

u1282+u2222=1

Answer. The principal directions are x2=x1 (at 45∘, eigenvalue 8, stretch factor 8) and x2=−x1 (at 135∘, eigenvalue 2, stretch factor 2). The circle is deformed into an ellipse with semi-axes 8 and 2 along these directions.

## Question 6: Numerical Differentiation

### (a) Rate of population growth

Question 6(a). The table below gives the census population (in millions) of a state for the years 1961 to 2001. Find the rate of growth of the population in the year 2001. (6 marks)

| Year (x) | 1961 | 1971 | 1981 | 1991 | 2001 |
| --- | --- | --- | --- | --- | --- |
| Population (y) | 19.96 | 36.65 | 58.81 | 77.21 | 94.61 |

The year 2001 is at the end of the table, so we use Newton’s backward difference formula with h=10 years.

Backward difference table.

| Year x | y | ∇y | ∇2y | ∇3y | ∇4y |
| --- | --- | --- | --- | --- | --- |
| 1961 | 19.96 |  |  |  |  |
| 1971 | 36.65 | 16.69 |  |  |  |
| 1981 | 58.81 | 22.16 | 5.47 |  |  |
| 1991 | 77.21 | 18.40 | −3.76 | −9.23 |  |
| 2001 | 94.61 | 17.40 | −1.00 | 2.76 | 11.99 |

Derivative at the last point xn=2001 (where p=0):

dydxxn=1h∇yn+12∇2yn+13∇3yn+14∇4yn

=11017.40+−1.002+2.763+11.994=11017.40−0.50+0.92+2.9975=20.817510=2.0818

Answer. In 2001 the population was growing at about 2.08 million per year.

### (b) Derivatives from tabulated data

Question 6(b). Given the following data points, determine dydx and d2ydx2 at x=1.1 and x=1.5. (6 marks)

| x | 1.0 | 1.1 | 1.2 | 1.3 | 1.4 | 1.5 | 1.6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| f(x) | 7.989 | 8.403 | 8.781 | 9.129 | 9.451 | 9.750 | 10.031 |

Here h=0.1. The point x=1.1 is near the start of the table, so we use Newton’s forward formula there; x=1.5 is near the end, so we use the backward formula.

Difference table (row xi lists Δkfi, the forward differences starting at that row).

| x | f(x) | Δ | Δ2 | Δ3 | Δ4 | Δ5 | Δ6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.0 | 7.989 | 0.414 | −0.036 | 0.006 | −0.002 | 0.001 | 0.002 |
| 1.1 | 8.403 | 0.378 | −0.030 | 0.004 | −0.001 | 0.003 |  |
| 1.2 | 8.781 | 0.348 | −0.026 | 0.003 | 0.002 |  |  |
| 1.3 | 9.129 | 0.322 | −0.023 | 0.005 |  |  |  |
| 1.4 | 9.451 | 0.299 | −0.018 |  |  |  |  |
| 1.5 | 9.750 | 0.281 |  |  |  |  |  |
| 1.6 | 10.031 |  |  |  |  |  |  |

### At x=1.1 (forward differences, x0=1.1)

From the table, Δy0=0.378, Δ2y0=−0.030, Δ3y0=0.004, Δ4y0=−0.001 and Δ5y0=0.003.

dydxx0=1hΔy0−12Δ2y0+13Δ3y0−14Δ4y0+15Δ5y0

=10.10.378+0.0302+0.0043+0.0014+0.0035=100.378+0.015+0.00133+0.00025+0.0006=3.952

d2ydx2x0=1h2Δ2y0−Δ3y0+1112Δ4y0−56Δ5y0

=100−0.030−0.004+1112(−0.001)−56(0.003)=100−0.030−0.004−0.00092−0.0025=−3.742

### At x=1.5 (backward differences, xn=1.5)

From the table, ∇yn=0.299, ∇2yn=−0.023, ∇3yn=0.003, ∇4yn=−0.001 and ∇5yn=0.001.

dydxxn=1h∇yn+12∇2yn+13∇3yn+14∇4yn+15∇5yn

=100.299−0.0115+0.001−0.00025+0.0002=2.885

d2ydx2xn=1h2∇2yn+∇3yn+1112∇4yn+56∇5yn

=100−0.023+0.003−0.00092+0.00083=−2.008

Answer. At x=1.1: dydx≈3.952 and d2ydx2≈−3.742. At x=1.5: dydx≈2.885 and d2ydx2≈−2.008.

## Question 7: Legendre’s Equation and the Wave Equation

### (a) Legendre’s equation

Question 7(a). Solve this special Legendre equation, (1−x2)y″−2xy′+2y=0, which occurs in models exhibiting spherical symmetry. (6 marks)

This is Legendre’s equation (1−x2)y″−2xy′+n(n+1)y=0 with n(n+1)=2, that is n=1.

Power series. Put y=m=0∞amxm. Then

(1−x2)m​m(m−1)amxm−2−2xm​mamxm−1+2m​amxm=0

Collecting the coefficient of xm:

(m+2)(m+1)am+2−m(m−1)+2m−2am=0

Since m(m−1)+2m−2=m2+m−2=(m+2)(m−1), the recurrence relation is

am+2=(m+2)(m−1)(m+2)(m+1) am=m−1m+1 am

Odd series (m=1): a3=0⋅a1=0, so a5=a7=⋯=0 and the series stops:

y1=a1x=a1P1(x)

Even series (from a0):

a2=−a0,  a4=13a2=−a03,  a6=35a4=−a05,  a8=−a07, …

y2=a01−x2−x43−x65−x87−⋯=a01−x2ln1+x1−x,  |x|<1

because 12ln1+x1−x=x+x33+x55+⋯.

Check of y1=x:  (1−x2)(0)−2x(1)+2x=0. ✓

Answer. y=c1x+c21−x2−x43−x65−⋯=c1P1(x)+c21−x2ln1+x1−x for x<1. The polynomial solution is the Legendre polynomial P1(x)=x.

### (b) One-dimensional wave equation

Question 7(b). Find the solution to this one-dimensional wave equation:  ∂2u∂x2=116∂2u∂t2 for 0<x<2, t>0. The boundary conditions are u(0,t)=u(2,t)=0. The initial conditions are (i) u(x,0)=6sinπx−3sin4πx and (ii) ∂u∂t(x,0)=0. (6 marks)

The equation is

∂2u∂x2=116∂2u∂t2 ⇔ ∂2u∂t2=c2∂2u∂x2,  c2=16, c=4, L=2

Separation of variables. Put u=F(x)G(t):

F″F=G″16 G=−p2

so F″+p2F=0 and G″+16p2G=0.

Boundary conditions. u(0,t)=u(2,t)=0 need F(0)=F(2)=0. With F=Acospx+Bsinpx, F(0)=0 gives A=0, and sin2p=0 gives

p=nπ2,  Fn(x)=sinnπx2,  n=1,2,3,…

Time part. Gn″+λn2Gn=0 with λn=cnπL=4nπ2=2nπ:

Gn(t)=Bncos2nπt+Bn*sin2nπt

so

u(x,t)=n=1∞Bncos2nπt+Bn*sin2nπtsinnπx2

Initial velocity. ∂u∂t(x,0)=∑2nπBn*sinnπx2=0, so Bn*=0 for every n.

Initial displacement.

u(x,0)=n=1∞Bnsinnπx2=6sinπx−3sin4πx

sinπx=sin2πx2 is the n=2 term and sin4πx=sin8πx2 is the n=8 term. Comparing coefficients (no Fourier integrals are needed):

B2=6,  B8=−3,  Bn=0 otherwise

With λ2=4π and λ8=16π:

Answer. u(x,t)=6sinπxcos4πt−3sin4πxcos16πt.

Check. u(0,t)=u(2,t)=0 since sin2π=sin8π=0; u(x,0)=6sinπx−3sin4πx; ut(x,0)=0; and each term satisfies utt=16uxx, for example (4π)2=16π2. ✓

# Type 3

## Question 2: Numerical Differentiation

### (a) Rate of population growth

Question 2(a). The table below gives the census population (in millions) of a state for the years 1961 to 2001. Find the rate of growth of the population in the year 2001. (6 marks)

| Year (x) | 1961 | 1971 | 1981 | 1991 | 2001 |
| --- | --- | --- | --- | --- | --- |
| Population (y) | 19.96 | 36.65 | 58.81 | 77.21 | 94.61 |

The year 2001 is at the end of the table, so we use Newton’s backward difference formula with h=10 years.

Backward difference table.

| Year x | y | ∇y | ∇2y | ∇3y | ∇4y |
| --- | --- | --- | --- | --- | --- |
| 1961 | 19.96 |  |  |  |  |
| 1971 | 36.65 | 16.69 |  |  |  |
| 1981 | 58.81 | 22.16 | 5.47 |  |  |
| 1991 | 77.21 | 18.40 | −3.76 | −9.23 |  |
| 2001 | 94.61 | 17.40 | −1.00 | 2.76 | 11.99 |

Derivative at the last point xn=2001 (where p=0):

dydxxn=1h∇yn+12∇2yn+13∇3yn+14∇4yn

=11017.40+−1.002+2.763+11.994=11017.40−0.50+0.92+2.9975=20.817510=2.0818

Answer. In 2001 the population was growing at about 2.08 million per year.

### (b) Derivatives from tabulated data

Question 2(b). Given the following data points, determine dydx and d2ydx2 at x=1.1 and x=1.5. (6 marks)

| x | 1.0 | 1.1 | 1.2 | 1.3 | 1.4 | 1.5 | 1.6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| f(x) | 7.989 | 8.403 | 8.781 | 9.129 | 9.451 | 9.750 | 10.031 |

Here h=0.1. The point x=1.1 is near the start of the table, so we use Newton’s forward formula there; x=1.5 is near the end, so we use the backward formula.

Difference table (row xi lists Δkfi, the forward differences starting at that row).

| x | f(x) | Δ | Δ2 | Δ3 | Δ4 | Δ5 | Δ6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.0 | 7.989 | 0.414 | −0.036 | 0.006 | −0.002 | 0.001 | 0.002 |
| 1.1 | 8.403 | 0.378 | −0.030 | 0.004 | −0.001 | 0.003 |  |
| 1.2 | 8.781 | 0.348 | −0.026 | 0.003 | 0.002 |  |  |
| 1.3 | 9.129 | 0.322 | −0.023 | 0.005 |  |  |  |
| 1.4 | 9.451 | 0.299 | −0.018 |  |  |  |  |
| 1.5 | 9.750 | 0.281 |  |  |  |  |  |
| 1.6 | 10.031 |  |  |  |  |  |  |

### At x=1.1 (forward differences, x0=1.1)

From the table, Δy0=0.378, Δ2y0=−0.030, Δ3y0=0.004, Δ4y0=−0.001 and Δ5y0=0.003.

dydxx0=1hΔy0−12Δ2y0+13Δ3y0−14Δ4y0+15Δ5y0

=10.10.378+0.0302+0.0043+0.0014+0.0035=100.378+0.015+0.00133+0.00025+0.0006=3.952

d2ydx2x0=1h2Δ2y0−Δ3y0+1112Δ4y0−56Δ5y0

=100−0.030−0.004+1112(−0.001)−56(0.003)=100−0.030−0.004−0.00092−0.0025=−3.742

### At x=1.5 (backward differences, xn=1.5)

From the table, ∇yn=0.299, ∇2yn=−0.023, ∇3yn=0.003, ∇4yn=−0.001 and ∇5yn=0.001.

dydxxn=1h∇yn+12∇2yn+13∇3yn+14∇4yn+15∇5yn

=100.299−0.0115+0.001−0.00025+0.0002=2.885

d2ydx2xn=1h2∇2yn+∇3yn+1112∇4yn+56∇5yn

=100−0.023+0.003−0.00092+0.00083=−2.008

Answer. At x=1.1: dydx≈3.952 and d2ydx2≈−3.742. At x=1.5: dydx≈2.885 and d2ydx2≈−2.008.

## Question 3: RLC Circuit

Question 3. Consider a series RLC circuit consisting of a 20 Ω resistor, a 1 H inductor and a 2000 μF capacitor. The circuit starts with zero initial charge and zero initial current. (a) Determine the expression for the charge on the capacitor at time t (i) when the circuit is connected to a constant 12 V DC source, and (ii) when the circuit is driven by an AC source with voltage E(t)=12sin10t. (6 marks) (b) For each case in part (a), also find the corresponding current in the circuit as a function of time. (6 marks)

The circuit equation. By Kirchhoff’s voltage law, with I=dq/dt,

Ld2qdt2+Rdqdt+qC=E(t)

Here R=20 Ω, L=1 H and C=2000 μF=2×10−3 F, so 1/C=500:

d2qdt2+20dqdt+500q=E(t),  q(0)=0, I(0)=q′(0)=0

Complementary function. r2+20r+500=0 gives r=−10±100−500=−10±20i:

qc=e−10tc1cos20t+c2sin20t

### Case (i): 12 V battery, E=12

Particular integral. Try a constant: 500qp=12, so qp=12500=0.024.

q=e−10tc1cos20t+c2sin20t+0.024

q(0)=0:  c1+0.024=0⇒c1=−0.024.

q′(0)=0:  −10c1+20c2=0⇒c2=12c1=−0.012.

q(t)=0.024−e−10t0.024cos20t+0.012sin20t=32502−e−10t2cos20t+sin20t

Current. Differentiating,

I=dqdt=10e−10t0.024cos20t+0.012sin20t−e−10t−0.48sin20t+0.24cos20t=0.6 e−10tsin20t

### Case (ii): generator, E=12sin10t

Particular integral. Try qp=Acos10t+Bsin10t, so qp″=−100qp:

(500−100)(Acos10t+Bsin10t)+20(−10Asin10t+10Bcos10t)=12sin10t

cos10t: 400A+200B=0,  sin10t: 400B−200A=12

From the first, B=−2A; then −800A−200A=12, so A=−0.012 and B=0.024:

qp=−0.012cos10t+0.024sin10t

Constants. q(0)=c1−0.012=0⇒c1=0.012.

q′(0)=−10c1+20c2+10(0.024)=−0.12+20c2+0.24=0⇒c2=−0.006.

q(t)=e−10t0.012cos20t−0.006sin20t−0.012cos10t+0.024sin10t

Current.

I=dqdt=−e−10t0.24cos20t+0.18sin20t+0.24cos10t+0.12sin10t

Check: I(0)=−0.24+0.24=0. ✓ The e−10t terms are the transient, which dies away; the steady-state current is 0.24cos10t+0.12sin10t.

Answer. (i) q=0.024−e−10t(0.024cos20t+0.012sin20t) C and I=0.6 e−10tsin20t A. (ii) q=e−10t(0.012cos20t−0.006sin20t)−0.012cos10t+0.024sin10t C and I=−e−10t(0.24cos20t+0.18sin20t)+0.24cos10t+0.12sin10t A.

## Question 4: Series Solutions and Bessel’s Equation

### (a) Power series solution

Question 4(a). Find the power series solution of 3(y″+y)=0. (6 marks)

Dividing by the non-zero constant 3 leaves y″+y=0.

Assume a power series solution about x=0:

y=n=0∞anxn,  y″=n=2∞n(n−1)anxn−2=n=0∞(n+2)(n+1)an+2 xn

Substituting into y″+y=0:

n=0∞(n+2)(n+1)an+2+anxn=0

Every coefficient must vanish, which gives the recurrence relation

an+2=−an(n+2)(n+1),  n=0,1,2,…

Even coefficients (in terms of a0):

a2=−a02!,  a4=−a24⋅3=a04!,  a6=−a46⋅5=−a06!, …

Odd coefficients (in terms of a1):

a3=−a13!,  a5=−a35⋅4=a15!,  a7=−a17!, …

Hence

y=a01−x22!+x44!−x66!+⋯+a1x−x33!+x55!−x77!+⋯

The two series are the Maclaurin series of cosx and sinx, and both converge for all x.

Answer. y=a01−x22!+x44!−⋯+a1x−x33!+x55!−⋯=a0cosx+a1sinx, where a0 and a1 are arbitrary constants.

### (b) Bessel’s equation

Question 4(b). Find the general solution, in terms of Bessel functions, of the equation  xy″+y′+36y=0. (6 marks)

The fraction in the equation reduces to 12, so the equation is

xy″+y′+12 y=0

Change of variable. Equations of the form xy″+y′+λy=0 become Bessel’s equation under z=2λx. Here λ=12, so put

z=2x2=2x,  z2=2x,  dzdx=1z

Then

dydx=1zdydz,  d2ydx2=1zddz1zdydz=1z2d2ydz2−1z3dydz

With x=z2/2:

xd2ydx2+dydx+12y=12d2ydz2−12zdydz+1zdydz+12y=12d2ydz2+1zdydz+y=0

Multiplying by 2z2:

z2d2ydz2+zdydz+(z2−ν2)y=0 with ν=0

This is Bessel’s equation of order ν=0.

General solution. For order ν=0 the two independent solutions are J0(z) and the Bessel function of the second kind, Y0(z):

y=c1J0(z)+c2Y0(z)=c1J0​2x+c2Y0​2x

In series form, the first solution is

J0​2x=m=0∞(−1)m(m!)2x2m=1−x2+x216−x3288+⋯

Answer. y=c1J0​2x+c2Y0​2x.

## Question 5: Legendre’s Equation and the Wave Equation

### (a) Legendre’s equation

Question 5(a). Solve this special Legendre equation, (1−x2)y″−2xy′+2y=0, which occurs in models exhibiting spherical symmetry. (6 marks)

This is Legendre’s equation (1−x2)y″−2xy′+n(n+1)y=0 with n(n+1)=2, that is n=1.

Power series. Put y=m=0∞amxm. Then

(1−x2)m​m(m−1)amxm−2−2xm​mamxm−1+2m​amxm=0

Collecting the coefficient of xm:

(m+2)(m+1)am+2−m(m−1)+2m−2am=0

Since m(m−1)+2m−2=m2+m−2=(m+2)(m−1), the recurrence relation is

am+2=(m+2)(m−1)(m+2)(m+1) am=m−1m+1 am

Odd series (m=1): a3=0⋅a1=0, so a5=a7=⋯=0 and the series stops:

y1=a1x=a1P1(x)

Even series (from a0):

a2=−a0,  a4=13a2=−a03,  a6=35a4=−a05,  a8=−a07, …

y2=a01−x2−x43−x65−x87−⋯=a01−x2ln1+x1−x,  |x|<1

because 12ln1+x1−x=x+x33+x55+⋯.

Check of y1=x:  (1−x2)(0)−2x(1)+2x=0. ✓

Answer. y=c1x+c21−x2−x43−x65−⋯=c1P1(x)+c21−x2ln1+x1−x for x<1. The polynomial solution is the Legendre polynomial P1(x)=x.

### (b) One-dimensional wave equation

Question 5(b). Find the solution to this one-dimensional wave equation:  ∂2u∂x2=116∂2u∂t2 for 0<x<2, t>0. The boundary conditions are u(0,t)=u(2,t)=0. The initial conditions are (i) u(x,0)=6sinπx−3sin4πx and (ii) ∂u∂t(x,0)=0. (6 marks)

The equation is

∂2u∂x2=116∂2u∂t2 ⇔ ∂2u∂t2=c2∂2u∂x2,  c2=16, c=4, L=2

Separation of variables. Put u=F(x)G(t):

F″F=G″16 G=−p2

so F″+p2F=0 and G″+16p2G=0.

Boundary conditions. u(0,t)=u(2,t)=0 need F(0)=F(2)=0. With F=Acospx+Bsinpx, F(0)=0 gives A=0, and sin2p=0 gives

p=nπ2,  Fn(x)=sinnπx2,  n=1,2,3,…

Time part. Gn″+λn2Gn=0 with λn=cnπL=4nπ2=2nπ:

Gn(t)=Bncos2nπt+Bn*sin2nπt

so

u(x,t)=n=1∞Bncos2nπt+Bn*sin2nπtsinnπx2

Initial velocity. ∂u∂t(x,0)=∑2nπBn*sinnπx2=0, so Bn*=0 for every n.

Initial displacement.

u(x,0)=n=1∞Bnsinnπx2=6sinπx−3sin4πx

sinπx=sin2πx2 is the n=2 term and sin4πx=sin8πx2 is the n=8 term. Comparing coefficients (no Fourier integrals are needed):

B2=6,  B8=−3,  Bn=0 otherwise

With λ2=4π and λ8=16π:

Answer. u(x,t)=6sinπxcos4πt−3sin4πxcos16πt.

Check. u(0,t)=u(2,t)=0 since sin2π=sin8π=0; u(x,0)=6sinπx−3sin4πx; ut(x,0)=0; and each term satisfies utt=16uxx, for example (4π)2=16π2. ✓

## Question 6: Eigenvalues and Eigenvectors

### (a) Eigenvalues and eigenvectors of a 3 × 3 matrix

Question 6(a). Find the eigenvalues and eigenvectors of the matrix (6 marks)

A=8−62−67−42−43

Characteristic equation. Expanding det(A−λI) along the first row,

(8−λ)(7−λ)(3−λ)−16+6−6(3−λ)+8+224−2(7−λ)=0

(8−λ)(λ2−10λ+5)+6(6λ−10)+2(2λ+10)=−λ3+18λ2−45λ=0

λ(λ2−18λ+45)=λ(λ−3)(λ−15)=0 ⇒ λ=0, 3, 15

As a check, the eigenvalues add up to the trace: 0+3+15=8+7+3=18. ✓

Eigenvectors.

λ=0:  8−62−67−42−43x=0. The cross product of the first two rows, (8,−6,2)×(−6,7,−4)=(10,20,20), gives x=(1,2,2)T.

λ=3:  5−62−64−42−40x=0. The third row gives x1=2x2; then the first gives 10x2−6x2+2x3=0, so x3=−2x2 and x=(2,1,−2)T.

λ=15:  −7−62−6−8−42−4−12x=0. The cross product of the first two rows, (−7,−6,2)×(−6,−8,−4)=(40,−40,20), gives x=(2,−2,1)T.

Check for λ=15: A(2,−2,1)T=(16+12+2, −12−14−4, 4+8+3)T=(30,−30,15)T=15(2,−2,1)T ✓. The matrix is symmetric, so the eigenvectors are mutually orthogonal.

Answer. λ1=0 with x1=k(1,2,2)T;  λ2=3 with x2=k(2,1,−2)T;  λ3=15 with x3=k(2,−2,1)T (k≠0).

### (b) Principal directions of a stretched membrane

Question 6(b). An elastic membrane in the x1x2-plane with boundary circle x12+x22=1 is stretched so that a point P:(x1,x2) goes over into the point Q:(y1,y2) given by

y=y1y2=Ax=5335x1x2

Find the principal directions, that is, the directions of the position vector x of P for which the direction of the position vector y of Q is the same or exactly opposite. (6 marks)

We need the vectors x for which y=Ax is parallel to x, that is Ax=λx. These are the eigenvectors of

A=5335

Eigenvalues.

det(A−λI)=(5−λ)2−9=λ2−10λ+16=(λ−8)(λ−2)=0 ⇒ λ1=8, λ2=2

Eigenvectors.

For λ1=8:  −3x1+3x2=0 ⇒ x2=x1, so x(1)=11, at 45∘ to the x1-axis.

For λ2=2:  3x1+3x2=0 ⇒ x2=−x1, so x(2)=1−1, at 135∘ (or −45∘) to the x1-axis.

Both eigenvalues are positive, so in each principal direction y points the same way as x. The two directions are perpendicular because A is symmetric.

Interpretation. Along the 45∘ direction the membrane is stretched by a factor of 8, and along the 135∘ direction by a factor of 2. With coordinates u1,u2 along these principal directions, the boundary circle x12+x22=1 becomes the ellipse

u1282+u2222=1

Answer. The principal directions are x2=x1 (at 45∘, eigenvalue 8, stretch factor 8) and x2=−x1 (at 135∘, eigenvalue 2, stretch factor 2). The circle is deformed into an ellipse with semi-axes 8 and 2 along these directions.

## Question 7: Differential Equations and Free Vibration

### (a) Non-homogeneous second-order ODE

Question 7(a). Find the solution to the engineering problem represented by the differential equation below. (6 marks)

d2ydx2+2dydx+5y=x2+xe2x+e−xsin2x

Complementary function.

m2+2m+5=0 ⇒ m=−1±2i,  yc=e−xAcos2x+Bsin2x

Particular integral. yp=y1+y2+y3.

For x2. Try y1=ax2+bx+c:

2a+2(2ax+b)+5(ax2+bx+c)=5ax2+(4a+5b)x+(2a+2b+5c)=x2

a=15, b=−4a5=−425, c=−2a+2b5=−2125

For xe2x. Put y2=u e2x:  y2″+2y2′+5y2=u″+6u′+13ue2x=xe2x. Try u=px+q:  6p+13(px+q)=x⇒p=113, q=−6169:

y2=13x−6169e2x

For e−xsin2x. This term has the same form as yc (since −1±2i are the roots), so an extra factor of x is needed. Put y3=v e−x; then y3′=(v′−v)e−x, y3″=(v″−2v′+v)e−x and

y3″+2y3′+5y3=v″−2v′+v+2v′−2v+5ve−x=v″+4ve−x=e−xsin2x

Try v=x(Ccos2x+Dsin2x). Then v″+4v=2ddx(Ccos2x+Dsin2x)=−4Csin2x+4Dcos2x=sin2x, so C=−14, D=0:

y3=−x4e−xcos2x

Answer. y=e−xAcos2x+Bsin2x+x25−4x25−2125+13x−6169e2x−x4e−xcos2x

### (b) Undamped spring–mass system

Question 7(b). A spring with a mass of 2000 g has natural length of one-half a meter. A force of 25.6 N is required to maintain it stretched to a length of 700 mm. If the spring is stretched to a length of 700 mm and then released with initial velocity 0 m/s: (i) What will be the position of the mass at any time, t? (ii) What length of travel (in m) would the mass achieve in t=π/24 seconds? (6 marks)

Spring constant. Hooke’s law, F=kx, with the extension x=0.7−0.5=0.2 m:

k(0.2)=25.6 ⇒ k=128 N/m

The mass is m=2000 g=2 kg. Let x(t) be the displacement of the mass from equilibrium (the natural length, 0.5 m), positive in the direction of stretching.

Equation of motion. There is no damping, so

md2xdt2+kx=0 ⇒ 2d2xdt2+128x=0 ⇒ d2xdt2+64x=0

The auxiliary equation r2+64=0 gives r=±8i, so

x(t)=c1cos8t+c2sin8t

Initial conditions. The spring starts stretched to 0.7 m, so x(0)=0.2, and it is released from rest, so x′(0)=0.

x(0)=c1=0.2,  x′(0)=8c2=0 ⇒ c2=0

(i) Position at any time.

x(t)=0.2cos8t m

The mass oscillates about the equilibrium position with amplitude 0.2 m, angular frequency ω=8 rad/s and period T=2π/8=π/4 s.

(ii) At t=π/24 s.

xπ24=0.2cos8π24=0.2cosπ3=0.2×0.5=0.1 m

The velocity x′(t)=−1.6sin8t stays negative for 0<t<π/8, and π/24<π/8, so the mass moves steadily back towards equilibrium without turning. The distance travelled from the start is therefore

0.2−0.1=0.1 m

and the spring’s length at that moment is 0.5+0.1=0.6 m.

Answer. (i) x(t)=0.2cos8t m. (ii) At t=π/24 s the mass is 0.1 m from equilibrium, having travelled 0.1 m from its starting point (spring length 0.6 m).

# Type 4

## Question 2: Differential Equations and Free Vibration

### (a) Non-homogeneous second-order ODE

Question 2(a). An engineering problem is represented by the differential equation below. Give the solution to the problem. (6 marks)

d2ydx2+4dydx+4y=x2+xe2x+e−2xsin2x

Complementary function.

m2+4m+4=(m+2)2=0 ⇒ m=−2 (repeated),  yc=(A+Bx)e−2x

Particular integral. yp=y1+y2+y3.

For x2. Try y1=ax2+bx+c:

2a+4(2ax+b)+4(ax2+bx+c)=4ax2+(8a+4b)x+(2a+4b+4c)=x2

a=14, b=−2a=−12, c=−2a+4b4=38

For xe2x. Put y2=u e2x:  y2″+4y2′+4y2=u″+8u′+16ue2x=xe2x. Try u=px+q:  8p+16(px+q)=x⇒p=116, q=−132:

y2=2x−132e2x

For e−2xsin2x. Put y3=v e−2x. Because −2 is a double root of the auxiliary equation, the operator reduces to v″:

y3″+4y3′+4y3=v″−4v′+4v+4v′−8v+4ve−2x=v″e−2x=e−2xsin2x

So v″=sin2x, v=−14sin2x and y3=−14e−2xsin2x.

Answer. y=(A+Bx)e−2x+x24−x2+38+2x−132e2x−14e−2xsin2x

### (b) Undamped spring–mass system

Question 2(b). A spring with a mass of 2000 g has natural length of one-half a meter. A force of 25.6 N is required to maintain it stretched to a length of 700 mm. If the spring is stretched to a length of 700 mm and then released with initial velocity 0 m/s: (i) What will be the position of the mass at any time, t? (ii) What length of travel (in m) would the mass achieve in t=π/24 seconds? (6 marks)

Spring constant. Hooke’s law, F=kx, with the extension x=0.7−0.5=0.2 m:

k(0.2)=25.6 ⇒ k=128 N/m

The mass is m=2000 g=2 kg. Let x(t) be the displacement of the mass from equilibrium (the natural length, 0.5 m), positive in the direction of stretching.

Equation of motion. There is no damping, so

md2xdt2+kx=0 ⇒ 2d2xdt2+128x=0 ⇒ d2xdt2+64x=0

The auxiliary equation r2+64=0 gives r=±8i, so

x(t)=c1cos8t+c2sin8t

Initial conditions. The spring starts stretched to 0.7 m, so x(0)=0.2, and it is released from rest, so x′(0)=0.

x(0)=c1=0.2,  x′(0)=8c2=0 ⇒ c2=0

(i) Position at any time.

x(t)=0.2cos8t m

The mass oscillates about the equilibrium position with amplitude 0.2 m, angular frequency ω=8 rad/s and period T=2π/8=π/4 s.

(ii) At t=π/24 s.

xπ24=0.2cos8π24=0.2cosπ3=0.2×0.5=0.1 m

The velocity x′(t)=−1.6sin8t stays negative for 0<t<π/8, and π/24<π/8, so the mass moves steadily back towards equilibrium without turning. The distance travelled from the start is therefore

0.2−0.1=0.1 m

and the spring’s length at that moment is 0.5+0.1=0.6 m.

Answer. (i) x(t)=0.2cos8t m. (ii) At t=π/24 s the mass is 0.1 m from equilibrium, having travelled 0.1 m from its starting point (spring length 0.6 m).

## Question 3: Numerical Differentiation

### (a) Rate of population growth

Question 3(a). The table below gives the census population (in millions) of a state for the years 1961 to 2001. Find the rate of growth of the population in the year 2001. (6 marks)

| Year (x) | 1961 | 1971 | 1981 | 1991 | 2001 |
| --- | --- | --- | --- | --- | --- |
| Population (y) | 19.96 | 36.65 | 58.81 | 77.21 | 94.61 |

The year 2001 is at the end of the table, so we use Newton’s backward difference formula with h=10 years.

Backward difference table.

| Year x | y | ∇y | ∇2y | ∇3y | ∇4y |
| --- | --- | --- | --- | --- | --- |
| 1961 | 19.96 |  |  |  |  |
| 1971 | 36.65 | 16.69 |  |  |  |
| 1981 | 58.81 | 22.16 | 5.47 |  |  |
| 1991 | 77.21 | 18.40 | −3.76 | −9.23 |  |
| 2001 | 94.61 | 17.40 | −1.00 | 2.76 | 11.99 |

Derivative at the last point xn=2001 (where p=0):

dydxxn=1h∇yn+12∇2yn+13∇3yn+14∇4yn

=11017.40+−1.002+2.763+11.994=11017.40−0.50+0.92+2.9975=20.817510=2.0818

Answer. In 2001 the population was growing at about 2.08 million per year.

### (b) Derivatives from tabulated data

Question 3(b). Given the following data points, determine dydx and d2ydx2 at x=1.1 and x=1.5. (6 marks)

| x | 1.0 | 1.1 | 1.2 | 1.3 | 1.4 | 1.5 | 1.6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| f(x) | 7.989 | 8.403 | 8.781 | 9.129 | 9.451 | 9.750 | 10.031 |

Here h=0.1. The point x=1.1 is near the start of the table, so we use Newton’s forward formula there; x=1.5 is near the end, so we use the backward formula.

Difference table (row xi lists Δkfi, the forward differences starting at that row).

| x | f(x) | Δ | Δ2 | Δ3 | Δ4 | Δ5 | Δ6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.0 | 7.989 | 0.414 | −0.036 | 0.006 | −0.002 | 0.001 | 0.002 |
| 1.1 | 8.403 | 0.378 | −0.030 | 0.004 | −0.001 | 0.003 |  |
| 1.2 | 8.781 | 0.348 | −0.026 | 0.003 | 0.002 |  |  |
| 1.3 | 9.129 | 0.322 | −0.023 | 0.005 |  |  |  |
| 1.4 | 9.451 | 0.299 | −0.018 |  |  |  |  |
| 1.5 | 9.750 | 0.281 |  |  |  |  |  |
| 1.6 | 10.031 |  |  |  |  |  |  |

### At x=1.1 (forward differences, x0=1.1)

From the table, Δy0=0.378, Δ2y0=−0.030, Δ3y0=0.004, Δ4y0=−0.001 and Δ5y0=0.003.

dydxx0=1hΔy0−12Δ2y0+13Δ3y0−14Δ4y0+15Δ5y0

=10.10.378+0.0302+0.0043+0.0014+0.0035=100.378+0.015+0.00133+0.00025+0.0006=3.952

d2ydx2x0=1h2Δ2y0−Δ3y0+1112Δ4y0−56Δ5y0

=100−0.030−0.004+1112(−0.001)−56(0.003)=100−0.030−0.004−0.00092−0.0025=−3.742

### At x=1.5 (backward differences, xn=1.5)

From the table, ∇yn=0.299, ∇2yn=−0.023, ∇3yn=0.003, ∇4yn=−0.001 and ∇5yn=0.001.

dydxxn=1h∇yn+12∇2yn+13∇3yn+14∇4yn+15∇5yn

=100.299−0.0115+0.001−0.00025+0.0002=2.885

d2ydx2xn=1h2∇2yn+∇3yn+1112∇4yn+56∇5yn

=100−0.023+0.003−0.00092+0.00083=−2.008

Answer. At x=1.1: dydx≈3.952 and d2ydx2≈−3.742. At x=1.5: dydx≈2.885 and d2ydx2≈−2.008.

## Question 4: Series Solutions and Bessel’s Equation

### (a) Power series solution

Question 4(a). Find the power series solution of 4(y″+y)=0. (6 marks)

Dividing by the non-zero constant 4 leaves y″+y=0.

Assume a power series solution about x=0:

y=n=0∞anxn,  y″=n=2∞n(n−1)anxn−2=n=0∞(n+2)(n+1)an+2 xn

Substituting into y″+y=0:

n=0∞(n+2)(n+1)an+2+anxn=0

Every coefficient must vanish, which gives the recurrence relation

an+2=−an(n+2)(n+1),  n=0,1,2,…

Even coefficients (in terms of a0):

a2=−a02!,  a4=−a24⋅3=a04!,  a6=−a46⋅5=−a06!, …

Odd coefficients (in terms of a1):

a3=−a13!,  a5=−a35⋅4=a15!,  a7=−a17!, …

Hence

y=a01−x22!+x44!−x66!+⋯+a1x−x33!+x55!−x77!+⋯

The two series are the Maclaurin series of cosx and sinx, and both converge for all x.

Answer. y=a01−x22!+x44!−⋯+a1x−x33!+x55!−⋯=a0cosx+a1sinx, where a0 and a1 are arbitrary constants.

### (b) Bessel’s equation

Question 4(b). Find the general solution, in terms of Bessel functions, of the equation  xy″+y′+48y=0. (6 marks)

The fraction in the equation reduces to 12, so the equation is

xy″+y′+12 y=0

Change of variable. Equations of the form xy″+y′+λy=0 become Bessel’s equation under z=2λx. Here λ=12, so put

z=2x2=2x,  z2=2x,  dzdx=1z

Then

dydx=1zdydz,  d2ydx2=1zddz1zdydz=1z2d2ydz2−1z3dydz

With x=z2/2:

xd2ydx2+dydx+12y=12d2ydz2−12zdydz+1zdydz+12y=12d2ydz2+1zdydz+y=0

Multiplying by 2z2:

z2d2ydz2+zdydz+(z2−ν2)y=0 with ν=0

This is Bessel’s equation of order ν=0.

General solution. For order ν=0 the two independent solutions are J0(z) and the Bessel function of the second kind, Y0(z):

y=c1J0(z)+c2Y0(z)=c1J0​2x+c2Y0​2x

In series form, the first solution is

J0​2x=m=0∞(−1)m(m!)2x2m=1−x2+x216−x3288+⋯

Answer. y=c1J0​2x+c2Y0​2x.

## Question 5: Eigenvalues and Eigenvectors

### (a) Eigenvalues and eigenvectors of a 3 × 3 matrix

Question 5(a). Find the eigenvalues and eigenvectors of the matrix (6 marks)

A=11−2−101−211

Characteristic equation. Expanding det(A−λI) along the first row,

(1−λ)−λ(1−λ)−1−1−(1−λ)+2−2−1−2λ

=(1−λ)(λ2−λ−1)−(1+λ)+2+4λ=−λ3+2λ2+3λ=0

λ(λ2−2λ−3)=λ(λ−3)(λ+1)=0 ⇒ λ=−1, 0, 3

As a check, −1+0+3=2=1+0+1, the trace. ✓

Eigenvectors.

λ=−1:  21−2−111−212x=0. Adding rows 1 and 3 gives 2x2=0, so x2=0 and x1=x3:  x=(1,0,1)T.

λ=0:  11−2−101−211x=0. Row 2 gives x1=x3; row 1 then gives x2=x3:  x=(1,1,1)T.

λ=3:  −21−2−1−31−21−2x=0. Row 1 gives x2=2x1+2x3; row 2 then gives −x1−6x1−6x3+x3=0, so 7x1=−5x3. Taking x3=−7:  x=(5,−4,−7)T.

Check for λ=3: A(5,−4,−7)T=(5−4+14, −5−7, −10−4−7)T=(15,−12,−21)T=3(5,−4,−7)T ✓

Answer. λ1=−1 with x1=k(1,0,1)T;  λ2=0 with x2=k(1,1,1)T;  λ3=3 with x3=k(5,−4,−7)T (k≠0).

### (b) Principal directions of a stretched membrane

Question 5(b). An elastic membrane in the x1x2-plane with boundary circle x12+x22=1 is stretched so that a point P:(x1,x2) goes over into the point Q:(y1,y2) given by

y=y1y2=Ax=5335x1x2

Find the principal directions, that is, the directions of the position vector x of P for which the direction of the position vector y of Q is the same or exactly opposite. (6 marks)

We need the vectors x for which y=Ax is parallel to x, that is Ax=λx. These are the eigenvectors of

A=5335

Eigenvalues.

det(A−λI)=(5−λ)2−9=λ2−10λ+16=(λ−8)(λ−2)=0 ⇒ λ1=8, λ2=2

Eigenvectors.

For λ1=8:  −3x1+3x2=0 ⇒ x2=x1, so x(1)=11, at 45∘ to the x1-axis.

For λ2=2:  3x1+3x2=0 ⇒ x2=−x1, so x(2)=1−1, at 135∘ (or −45∘) to the x1-axis.

Both eigenvalues are positive, so in each principal direction y points the same way as x. The two directions are perpendicular because A is symmetric.

Interpretation. Along the 45∘ direction the membrane is stretched by a factor of 8, and along the 135∘ direction by a factor of 2. With coordinates u1,u2 along these principal directions, the boundary circle x12+x22=1 becomes the ellipse

u1282+u2222=1

Answer. The principal directions are x2=x1 (at 45∘, eigenvalue 8, stretch factor 8) and x2=−x1 (at 135∘, eigenvalue 2, stretch factor 2). The circle is deformed into an ellipse with semi-axes 8 and 2 along these directions.

## Question 6: Legendre’s Equation and the Wave Equation

### (a) Legendre’s equation

Question 6(a). Solve this special Legendre equation, (1−x2)y″−2xy′+2y=0, which occurs in models exhibiting spherical symmetry. (6 marks)

This is Legendre’s equation (1−x2)y″−2xy′+n(n+1)y=0 with n(n+1)=2, that is n=1.

Power series. Put y=m=0∞amxm. Then

(1−x2)m​m(m−1)amxm−2−2xm​mamxm−1+2m​amxm=0

Collecting the coefficient of xm:

(m+2)(m+1)am+2−m(m−1)+2m−2am=0

Since m(m−1)+2m−2=m2+m−2=(m+2)(m−1), the recurrence relation is

am+2=(m+2)(m−1)(m+2)(m+1) am=m−1m+1 am

Odd series (m=1): a3=0⋅a1=0, so a5=a7=⋯=0 and the series stops:

y1=a1x=a1P1(x)

Even series (from a0):

a2=−a0,  a4=13a2=−a03,  a6=35a4=−a05,  a8=−a07, …

y2=a01−x2−x43−x65−x87−⋯=a01−x2ln1+x1−x,  |x|<1

because 12ln1+x1−x=x+x33+x55+⋯.

Check of y1=x:  (1−x2)(0)−2x(1)+2x=0. ✓

Answer. y=c1x+c21−x2−x43−x65−⋯=c1P1(x)+c21−x2ln1+x1−x for x<1. The polynomial solution is the Legendre polynomial P1(x)=x.

### (b) One-dimensional wave equation

Question 6(b). Find the solution to this one-dimensional wave equation:  ∂2u∂x2=116∂2u∂t2 for 0<x<2, t>0. The boundary conditions are u(0,t)=u(2,t)=0. The initial conditions are (i) u(x,0)=6sinπx−3sin4πx and (ii) ∂u∂t(x,0)=0. (6 marks)

The equation is

∂2u∂x2=116∂2u∂t2 ⇔ ∂2u∂t2=c2∂2u∂x2,  c2=16, c=4, L=2

Separation of variables. Put u=F(x)G(t):

F″F=G″16 G=−p2

so F″+p2F=0 and G″+16p2G=0.

Boundary conditions. u(0,t)=u(2,t)=0 need F(0)=F(2)=0. With F=Acospx+Bsinpx, F(0)=0 gives A=0, and sin2p=0 gives

p=nπ2,  Fn(x)=sinnπx2,  n=1,2,3,…

Time part. Gn″+λn2Gn=0 with λn=cnπL=4nπ2=2nπ:

Gn(t)=Bncos2nπt+Bn*sin2nπt

so

u(x,t)=n=1∞Bncos2nπt+Bn*sin2nπtsinnπx2

Initial velocity. ∂u∂t(x,0)=∑2nπBn*sinnπx2=0, so Bn*=0 for every n.

Initial displacement.

u(x,0)=n=1∞Bnsinnπx2=6sinπx−3sin4πx

sinπx=sin2πx2 is the n=2 term and sin4πx=sin8πx2 is the n=8 term. Comparing coefficients (no Fourier integrals are needed):

B2=6,  B8=−3,  Bn=0 otherwise

With λ2=4π and λ8=16π:

Answer. u(x,t)=6sinπxcos4πt−3sin4πxcos16πt.

Check. u(0,t)=u(2,t)=0 since sin2π=sin8π=0; u(x,0)=6sinπx−3sin4πx; ut(x,0)=0; and each term satisfies utt=16uxx, for example (4π)2=16π2. ✓

## Question 7: RLC Circuit

Question 7. Consider an RLC circuit with a 1750 Ω resistor, a 1100 H inductor and a 10093 F capacitor, driven by the voltage E(t)=0.06sin3t V. (a) Write the differential equation associated with this circuit in terms of the current I. (6 marks) (b) If the initial charge and initial current on the capacitor are both zero, find the current I and the voltage across the resistor ER in terms of time t. (6 marks)

### (a) Differential equation in terms of I

Kirchhoff’s voltage law, with q=∫I dt, gives

LdIdt+RI+1C∫I dt=E(t)

Differentiating with respect to t removes the integral:

Ld2Idt2+RdIdt+1CI=dEdt

With L=1100, R=1750, 1C=93100 and dEdt=0.18cos3t:

1100d2Idt2+1750dIdt+93100I=0.18cos3t

Multiplying by 100:

d2Idt2+34dIdt+93I=18cos3t

### (b) Current and resistor voltage

Initial conditions. I(0)=0. At t=0 the circuit equation gives LI′(0)=E(0)−RI(0)−q(0)/C=0−0−0, so I′(0)=0.

Complementary function. r2+34r+93=0 gives r=−34±1156−3722=−34±282, so r=−3 or r=−31:

Ic=c1e−3t+c2e−31t

Particular integral. Try Ip=Acos3t+Bsin3t, with Ip″=−9Ip:

(93−9)(Acos3t+Bsin3t)+34(−3Asin3t+3Bcos3t)=18cos3t

cos3t: 84A+102B=18,  sin3t: 84B−102A=0

The second gives B=1714A. Substituting, 84A+173414A=18, so 291014A=18, giving A=42485 and B=51485.

Constants.

I(0)=c1+c2+42485=0,  I′(0)=−3c1−31c2+153485=0

Solving: c1=−328 and c2=27913580.

Current.

I(t)=−328e−3t+27913580e−31t+42cos3t+51sin3t485

I(t)≈−0.1071e−3t+0.0205e−31t+0.0866cos3t+0.1052sin3t A

Voltage across the resistor. ER=RI=1750I:

ER(t)=−511400e−3t+4743679000e−31t+35712125cos3t+86724250sin3t

ER(t)≈−0.0364e−3t+0.0070e−31t+0.0294cos3t+0.0358sin3t V

Answer. (a) I″+34I′+93I=18cos3t. (b) I=−328e−3t+27913580e−31t+1485(42cos3t+51sin3t) A, and ER=1750I≈−0.0364e−3t+0.0070e−31t+0.0294cos3t+0.0358sin3t V.

# Type 5

## Question 2: Differential Equations and Free Vibration

### (a) Non-homogeneous second-order ODE

Question 2(a). Find the solution to the engineering problem represented by the differential equation below. (6 marks)

d2ydx2+2dydx+5y=x2+xe2x+e−xcos2x

Complementary function.

m2+2m+5=0 ⇒ m=−1±2i,  yc=e−xAcos2x+Bsin2x

Particular integral. yp=y1+y2+y3.

For x2. Try y1=ax2+bx+c:

5ax2+(4a+5b)x+(2a+2b+5c)=x2 ⇒ a=15, b=−425, c=−2125

For xe2x. Put y2=u e2x:  u″+6u′+13ue2x=xe2x. Try u=px+q:  6p+13(px+q)=x⇒p=113, q=−6169:

y2=13x−6169e2x

For e−xcos2x. This term has the same form as yc, so an extra factor of x is needed. Put y3=v e−x; then

y3″+2y3′+5y3=v″−2v′+v+2v′−2v+5ve−x=v″+4ve−x=e−xcos2x

Try v=x(Ccos2x+Dsin2x). Then v″+4v=2ddx(Ccos2x+Dsin2x)=−4Csin2x+4Dcos2x=cos2x, so C=0, D=14:

y3=x4e−xsin2x

Answer. y=e−xAcos2x+Bsin2x+x25−4x25−2125+13x−6169e2x+x4e−xsin2x

### (b) Undamped spring–mass system

Question 2(b). A spring with a mass of 2000 g has natural length of one-half a meter. A force of 25.6 N is required to maintain it stretched to a length of 700 mm. If the spring is stretched to a length of 700 mm and then released with initial velocity 0 m/s: (i) What will be the position of the mass at any time, t? (ii) What length of travel (in m) would the mass achieve in t=π/24 seconds? (6 marks)

Spring constant. Hooke’s law, F=kx, with the extension x=0.7−0.5=0.2 m:

k(0.2)=25.6 ⇒ k=128 N/m

The mass is m=2000 g=2 kg. Let x(t) be the displacement of the mass from equilibrium (the natural length, 0.5 m), positive in the direction of stretching.

Equation of motion. There is no damping, so

md2xdt2+kx=0 ⇒ 2d2xdt2+128x=0 ⇒ d2xdt2+64x=0

The auxiliary equation r2+64=0 gives r=±8i, so

x(t)=c1cos8t+c2sin8t

Initial conditions. The spring starts stretched to 0.7 m, so x(0)=0.2, and it is released from rest, so x′(0)=0.

x(0)=c1=0.2,  x′(0)=8c2=0 ⇒ c2=0

(i) Position at any time.

x(t)=0.2cos8t m

The mass oscillates about the equilibrium position with amplitude 0.2 m, angular frequency ω=8 rad/s and period T=2π/8=π/4 s.

(ii) At t=π/24 s.

xπ24=0.2cos8π24=0.2cosπ3=0.2×0.5=0.1 m

The velocity x′(t)=−1.6sin8t stays negative for 0<t<π/8, and π/24<π/8, so the mass moves steadily back towards equilibrium without turning. The distance travelled from the start is therefore

0.2−0.1=0.1 m

and the spring’s length at that moment is 0.5+0.1=0.6 m.

Answer. (i) x(t)=0.2cos8t m. (ii) At t=π/24 s the mass is 0.1 m from equilibrium, having travelled 0.1 m from its starting point (spring length 0.6 m).

## Question 3: RLC Circuit

Question 3. A series RLC circuit consists of a 5 Ω resistor, a 0.1 H inductor and a 1000 μF capacitor. The circuit starts with zero charge on the capacitor and zero current through the inductor. (a) Find the charge on the capacitor as a function of time (i) when the circuit is connected to a 6 V DC source, and (ii) when the circuit is driven by an AC source given by E(t)=6sin2t. (6 marks) (b) For each case in part (a), determine the corresponding current in the circuit. (6 marks)

The circuit equation. With C=1000 μF=10−3 F, so 1/C=1000:

0.1d2qdt2+5dqdt+1000q=E(t) ⇒ d2qdt2+50dqdt+10000q=10E(t)

with q(0)=0 and q′(0)=I(0)=0.

Complementary function. r2+50r+10000=0 gives

r=−25±625−10000=−25±i9375=−25±2515 i

Write ω=2515≈96.82 rad/s. Then

qc=e−25tc1cosωt+c2sinωt

### Case (i): 6 V DC source

Particular integral. 10000qp=60, so qp=0.006.

Constants. q(0)=c1+0.006=0⇒c1=−0.006.  q′(0)=−25c1+ωc2=0⇒c2=25c1ω=−0.00615=−152500≈−0.001549.

q(t)=0.0061−e−25tcosωt+115sinωt

Current. Differentiating (the cosine terms cancel),

I(t)=0.006ω+625ωe−25tsinωt=60ωe−25tsinωt=41525e−25tsinωt≈0.6197 e−25tsin(96.82t)

### Case (ii): AC source, E=6sin2t

The right-hand side is 10E=60sin2t.

Particular integral. Try qp=Acos2t+Bsin2t, with qp″=−4qp:

cos2t: 9996A+100B=0,  sin2t: 9996B−100A=60

Solving,

B=60×999699962+1002=374856245626≈6.0018×10−3,  A=−100B9996=−3756245626≈−6.004×10−5

Constants. q(0)=c1+A=0⇒c1=6.004×10−5.  q′(0)=−25c1+ωc2+2B=0⇒c2=25c1−2Bω≈−1.0847×10−4.

q(t)≈e−25t6.004×10−5cosωt−1.0847×10−4sinωt+6.0018×10−3sin2t−6.004×10−5cos2t

Current.

I(t)≈−e−25t0.012004cosωt+0.003102sinωt+0.012004cos2t+0.000120sin2t

Check: I(0)=−0.012004+0.012004=0. ✓ The transient dies away within about 0.2 s (since e−25t is then below 1 %), leaving the steady-state current ≈0.0120cos2t A.

Answer. (i) q=0.0061−e−25tcos2515 t+115sin2515 t C and I=41525e−25tsin2515 t≈0.620 e−25tsin96.82t A. (ii) q≈e−25t6.00×10−5cos96.82t−1.08×10−4sin96.82t+6.00×10−3sin2t−6.00×10−5cos2t C and I≈−e−25t0.0120cos96.82t+0.0031sin96.82t+0.0120cos2t+0.00012sin2t A.

## Question 4: Numerical Differentiation

### (a) Rate of population growth

Question 4(a). The table below gives the census population (in millions) of a state for the years 1961 to 2001. Find the rate of growth of the population in the year 2001. (6 marks)

| Year (x) | 1961 | 1971 | 1981 | 1991 | 2001 |
| --- | --- | --- | --- | --- | --- |
| Population (y) | 19.96 | 36.65 | 58.81 | 77.21 | 94.61 |

The year 2001 is at the end of the table, so we use Newton’s backward difference formula with h=10 years.

Backward difference table.

| Year x | y | ∇y | ∇2y | ∇3y | ∇4y |
| --- | --- | --- | --- | --- | --- |
| 1961 | 19.96 |  |  |  |  |
| 1971 | 36.65 | 16.69 |  |  |  |
| 1981 | 58.81 | 22.16 | 5.47 |  |  |
| 1991 | 77.21 | 18.40 | −3.76 | −9.23 |  |
| 2001 | 94.61 | 17.40 | −1.00 | 2.76 | 11.99 |

Derivative at the last point xn=2001 (where p=0):

dydxxn=1h∇yn+12∇2yn+13∇3yn+14∇4yn

=11017.40+−1.002+2.763+11.994=11017.40−0.50+0.92+2.9975=20.817510=2.0818

Answer. In 2001 the population was growing at about 2.08 million per year.

### (b) Derivatives from tabulated data

Question 4(b). Given the following data points, determine dydx and d2ydx2 at x=1.1 and x=1.5. (6 marks)

| x | 1.0 | 1.1 | 1.2 | 1.3 | 1.4 | 1.5 | 1.6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| f(x) | 7.989 | 8.403 | 8.781 | 9.129 | 9.451 | 9.750 | 10.031 |

Here h=0.1. The point x=1.1 is near the start of the table, so we use Newton’s forward formula there; x=1.5 is near the end, so we use the backward formula.

Difference table (row xi lists Δkfi, the forward differences starting at that row).

| x | f(x) | Δ | Δ2 | Δ3 | Δ4 | Δ5 | Δ6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.0 | 7.989 | 0.414 | −0.036 | 0.006 | −0.002 | 0.001 | 0.002 |
| 1.1 | 8.403 | 0.378 | −0.030 | 0.004 | −0.001 | 0.003 |  |
| 1.2 | 8.781 | 0.348 | −0.026 | 0.003 | 0.002 |  |  |
| 1.3 | 9.129 | 0.322 | −0.023 | 0.005 |  |  |  |
| 1.4 | 9.451 | 0.299 | −0.018 |  |  |  |  |
| 1.5 | 9.750 | 0.281 |  |  |  |  |  |
| 1.6 | 10.031 |  |  |  |  |  |  |

### At x=1.1 (forward differences, x0=1.1)

From the table, Δy0=0.378, Δ2y0=−0.030, Δ3y0=0.004, Δ4y0=−0.001 and Δ5y0=0.003.

dydxx0=1hΔy0−12Δ2y0+13Δ3y0−14Δ4y0+15Δ5y0

=10.10.378+0.0302+0.0043+0.0014+0.0035=100.378+0.015+0.00133+0.00025+0.0006=3.952

d2ydx2x0=1h2Δ2y0−Δ3y0+1112Δ4y0−56Δ5y0

=100−0.030−0.004+1112(−0.001)−56(0.003)=100−0.030−0.004−0.00092−0.0025=−3.742

### At x=1.5 (backward differences, xn=1.5)

From the table, ∇yn=0.299, ∇2yn=−0.023, ∇3yn=0.003, ∇4yn=−0.001 and ∇5yn=0.001.

dydxxn=1h∇yn+12∇2yn+13∇3yn+14∇4yn+15∇5yn

=100.299−0.0115+0.001−0.00025+0.0002=2.885

d2ydx2xn=1h2∇2yn+∇3yn+1112∇4yn+56∇5yn

=100−0.023+0.003−0.00092+0.00083=−2.008

Answer. At x=1.1: dydx≈3.952 and d2ydx2≈−3.742. At x=1.5: dydx≈2.885 and d2ydx2≈−2.008.

## Question 5: Eigenvalues and Eigenvectors

### (a) Eigenvalues and eigenvectors of a 3 × 3 matrix

Question 5(a). Find the eigenvalues and eigenvectors of the matrix (6 marks)

A=4−430−19001

Eigenvalues. A is upper triangular, so

det(A−λI)=(4−λ)(−1−λ)(1−λ)=0 ⇒ λ=4, −1, 1

(Expanded: λ3−4λ2−λ+4=0.)

Eigenvectors.

λ=4:  0−430−5900−3x=0 ⇒ x3=0, x2=0, x1 free ⇒ x=(1,0,0)T

λ=−1:  5−43009002x=0 ⇒ x3=0, 5x1=4x2 ⇒ x=(4,5,0)T

λ=1:  3−430−29000x=0 ⇒ x2=92x3, 3x1=4x2−3x3=15x3⇒x1=5x3. Taking x3=2:  x=(10,9,2)T

Check for λ=1: A(10,9,2)T=(40−36+6, −9+18, 2)T=(10,9,2)T ✓

Answer. λ1=4 with x1=k(1,0,0)T;  λ2=−1 with x2=k(4,5,0)T;  λ3=1 with x3=k(10,9,2)T (k≠0).

### (b) Principal directions of a stretched membrane

Question 5(b). An elastic membrane in the x1x2-plane with boundary circle x12+x22=1 is stretched so that a point P:(x1,x2) goes over into the point Q:(y1,y2) given by

y=y1y2=Ax=5335x1x2

Find the principal directions, that is, the directions of the position vector x of P for which the direction of the position vector y of Q is the same or exactly opposite. (6 marks)

We need the vectors x for which y=Ax is parallel to x, that is Ax=λx. These are the eigenvectors of

A=5335

Eigenvalues.

det(A−λI)=(5−λ)2−9=λ2−10λ+16=(λ−8)(λ−2)=0 ⇒ λ1=8, λ2=2

Eigenvectors.

For λ1=8:  −3x1+3x2=0 ⇒ x2=x1, so x(1)=11, at 45∘ to the x1-axis.

For λ2=2:  3x1+3x2=0 ⇒ x2=−x1, so x(2)=1−1, at 135∘ (or −45∘) to the x1-axis.

Both eigenvalues are positive, so in each principal direction y points the same way as x. The two directions are perpendicular because A is symmetric.

Interpretation. Along the 45∘ direction the membrane is stretched by a factor of 8, and along the 135∘ direction by a factor of 2. With coordinates u1,u2 along these principal directions, the boundary circle x12+x22=1 becomes the ellipse

u1282+u2222=1

Answer. The principal directions are x2=x1 (at 45∘, eigenvalue 8, stretch factor 8) and x2=−x1 (at 135∘, eigenvalue 2, stretch factor 2). The circle is deformed into an ellipse with semi-axes 8 and 2 along these directions.

## Question 6: Series Solutions and Bessel’s Equation

### (a) Power series solution

Question 6(a). Find the power series solution of 5(y″+y)=0. (6 marks)

Dividing by the non-zero constant 5 leaves y″+y=0.

Assume a power series solution about x=0:

y=n=0∞anxn,  y″=n=2∞n(n−1)anxn−2=n=0∞(n+2)(n+1)an+2 xn

Substituting into y″+y=0:

n=0∞(n+2)(n+1)an+2+anxn=0

Every coefficient must vanish, which gives the recurrence relation

an+2=−an(n+2)(n+1),  n=0,1,2,…

Even coefficients (in terms of a0):

a2=−a02!,  a4=−a24⋅3=a04!,  a6=−a46⋅5=−a06!, …

Odd coefficients (in terms of a1):

a3=−a13!,  a5=−a35⋅4=a15!,  a7=−a17!, …

Hence

y=a01−x22!+x44!−x66!+⋯+a1x−x33!+x55!−x77!+⋯

The two series are the Maclaurin series of cosx and sinx, and both converge for all x.

Answer. y=a01−x22!+x44!−⋯+a1x−x33!+x55!−⋯=a0cosx+a1sinx, where a0 and a1 are arbitrary constants.

### (b) Bessel’s equation

Question 6(b). Find the general solution, in terms of Bessel functions, of the equation  xy″+y′+510y=0. (6 marks)

The fraction in the equation reduces to 12, so the equation is

xy″+y′+12 y=0

Change of variable. Equations of the form xy″+y′+λy=0 become Bessel’s equation under z=2λx. Here λ=12, so put

z=2x2=2x,  z2=2x,  dzdx=1z

Then

dydx=1zdydz,  d2ydx2=1zddz1zdydz=1z2d2ydz2−1z3dydz

With x=z2/2:

xd2ydx2+dydx+12y=12d2ydz2−12zdydz+1zdydz+12y=12d2ydz2+1zdydz+y=0

Multiplying by 2z2:

z2d2ydz2+zdydz+(z2−ν2)y=0 with ν=0

This is Bessel’s equation of order ν=0.

General solution. For order ν=0 the two independent solutions are J0(z) and the Bessel function of the second kind, Y0(z):

y=c1J0(z)+c2Y0(z)=c1J0​2x+c2Y0​2x

In series form, the first solution is

J0​2x=m=0∞(−1)m(m!)2x2m=1−x2+x216−x3288+⋯

Answer. y=c1J0​2x+c2Y0​2x.

## Question 7: Legendre’s Equation and the Wave Equation

### (a) Legendre’s equation

Question 7(a). Solve this special Legendre equation, (1−x2)y″−2xy′+2y=0, which occurs in models exhibiting spherical symmetry. (6 marks)

This is Legendre’s equation (1−x2)y″−2xy′+n(n+1)y=0 with n(n+1)=2, that is n=1.

Power series. Put y=m=0∞amxm. Then

(1−x2)m​m(m−1)amxm−2−2xm​mamxm−1+2m​amxm=0

Collecting the coefficient of xm:

(m+2)(m+1)am+2−m(m−1)+2m−2am=0

Since m(m−1)+2m−2=m2+m−2=(m+2)(m−1), the recurrence relation is

am+2=(m+2)(m−1)(m+2)(m+1) am=m−1m+1 am

Odd series (m=1): a3=0⋅a1=0, so a5=a7=⋯=0 and the series stops:

y1=a1x=a1P1(x)

Even series (from a0):

a2=−a0,  a4=13a2=−a03,  a6=35a4=−a05,  a8=−a07, …

y2=a01−x2−x43−x65−x87−⋯=a01−x2ln1+x1−x,  |x|<1

because 12ln1+x1−x=x+x33+x55+⋯.

Check of y1=x:  (1−x2)(0)−2x(1)+2x=0. ✓

Answer. y=c1x+c21−x2−x43−x65−⋯=c1P1(x)+c21−x2ln1+x1−x for x<1. The polynomial solution is the Legendre polynomial P1(x)=x.

### (b) One-dimensional wave equation

Question 7(b). Find the solution to this one-dimensional wave equation:  ∂2u∂x2=116∂2u∂t2 for 0<x<2, t>0. The boundary conditions are u(0,t)=u(2,t)=0. The initial conditions are (i) u(x,0)=6sinπx−3sin4πx and (ii) ∂u∂t(x,0)=0. (6 marks)

The equation is

∂2u∂x2=116∂2u∂t2 ⇔ ∂2u∂t2=c2∂2u∂x2,  c2=16, c=4, L=2

Separation of variables. Put u=F(x)G(t):

F″F=G″16 G=−p2

so F″+p2F=0 and G″+16p2G=0.

Boundary conditions. u(0,t)=u(2,t)=0 need F(0)=F(2)=0. With F=Acospx+Bsinpx, F(0)=0 gives A=0, and sin2p=0 gives

p=nπ2,  Fn(x)=sinnπx2,  n=1,2,3,…

Time part. Gn″+λn2Gn=0 with λn=cnπL=4nπ2=2nπ:

Gn(t)=Bncos2nπt+Bn*sin2nπt

so

u(x,t)=n=1∞Bncos2nπt+Bn*sin2nπtsinnπx2

Initial velocity. ∂u∂t(x,0)=∑2nπBn*sinnπx2=0, so Bn*=0 for every n.

Initial displacement.

u(x,0)=n=1∞Bnsinnπx2=6sinπx−3sin4πx

sinπx=sin2πx2 is the n=2 term and sin4πx=sin8πx2 is the n=8 term. Comparing coefficients (no Fourier integrals are needed):

B2=6,  B8=−3,  Bn=0 otherwise

With λ2=4π and λ8=16π:

Answer. u(x,t)=6sinπxcos4πt−3sin4πxcos16πt.

Check. u(0,t)=u(2,t)=0 since sin2π=sin8π=0; u(x,0)=6sinπx−3sin4πx; ut(x,0)=0; and each term satisfies utt=16uxx, for example (4π)2=16π2. ✓